3GPP TSG-SA3 Meeting #118	S3-244132

Hyderabad, India  14 - 18 October 2024

Title:	 Progress update on security handling for inter-CU LTM in non-DC cases

Response to:

Release:	Rel-19

Work Item:	NR_Mob_Ph4-Core

Source:	Qualcomm Incorporated (to be SA3)

To:	RAN2, RAN3

Cc:

Contact person:	Adrian Escott

aescott@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA3 would like to provide the RAN2 and RAN3 with the following progress update on their work on LTM. SA3 is continuing to study possible solutions for inter-CU LTM handovers. Overall it may be difficult for SA3 to make a final conclusion on the final security solution as SA3 can only analyse the security impact and not the overall impact. It may be that SA3 supply a set of possible solutions and leave it to the RAN2 and RAN3 to decide on the best approach as long as security requirements are met.

SA3 are working applying the following principles when studying solutions:

- If the PDCP anchor does not change then there is no need to rekey (i.e., the same principle that is applied to intra-CU LTM handovers and L3 handovers).

- When PDCP anchor changes, it is necessary to rekey. Further to this point, there is no agreement to relax the need to have at or provide to the target anchor point after the handover a key unknown to the source anchor point, for so-called forward security (despite SA3 discussing such solutions).

In the original LS, SA3 was asked the following question

RAN2 requests SA3 whether, for each option, the change of security algorithm or the change of key set indicator is to be supported for inter-CU LTM.

SA3 would like to provide the following answer:

The change of security algorithm needs to be supported.

The use of the change of key set indicator should be a rare event so this does not need to be supported in LTM handovers provided it is possible to fallback to existing handovers (e.g. L3) that allow this key to change to happen.

SA3 ask RAN2 and RAN3 to take the above information into account.

# 2	Actions

To RAN2 and RAN3

ACTION: 	SA3 asks RAN2 and RAN3 to take the above information into account.

# 3	Dates of next TSG SA WG 3 meetings

SA3#119	11 - 15 November 2024		Orlando (US)

SA3#120	17 - 21 February 2025		Athens (Greece)