3GPP TSG-SA3 Meeting #118	S3-244143

# Hyderabad, India, 14th - 18th October 2024

# Source:	Qualcomm Incorporated

# Title:	Conclusion of KI#2

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

This contribution proposes a conclusion of KI #2.

# 2	References

[1]	TR 33.743

# 3	Rationale

This contribution proposes to add a conclusion of Key Issue #2.

Particularly, for IP PDU type of muti-hop UE-to-UE relay, it is proposed to use the security procedures in solution #12, #14 and #15 as a basis for normative work.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR [1].

***** START OF CHANGES *****

## 7.x	Key Issue #2: Security for Multi-hop UE-to-UE Relay

The following statements are agreed as a basis for normative work:

For multi-hop UE-to-UE relay discovery of IP PDU type:

- The security procedures for both relay discovery among 5G ProSe UE-to-UE Relays and relay discovery between an 5G ProSe End UE and 5G ProSe UE-to-UE Relay are based on solution #14.

For multi-hop UE-to-UE relay communication traffic of IP PDU type:

- The security procedure for both PC5 link establishment among 5G ProSe UE-to-UE Relays and PC5 link establishment between an 5G ProSe End UE and 5G ProSe UE-to-UE Relay is based on solution #12 and solution #15.

***** END OF CHANGES *****