3GPP TSG-SA3 Meeting #118	S3-244145

Hyderabad, India, 14th - 18th October 2024

# Source:	Qualcomm Incorporated

# Title:	Updating evaluations to solution #7

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

This contribution proposes to update evaluations to solution #7.

# 2	References

[1]	TS 33.503

[2]	TR 33.743

# 3	Rationale

This contribution proposes to update the evaluation to solution #7.

Particularly, it proposes to capture deviations from the discovery security materials provisioning and security protection mechanism for 5G ProSe UE-to-Network Relay defined in TS 33.503 [1].

Specifically, for discovery with Model B, a UE-to-Network Relay needs to obtain multiple sets of discovery security materials associated with the HPLMN of Intermediate Relay(s)/Remote UE(s), which deviates from the discovery security materials provisioning procedure for 5G ProSe UE-to-Network Relay defined in TS 33.503 [1].

Additionally, the solution specifies that the Remote UE, Intermediate Relay, and UE-to-Network Relay protect the discovery message using the discovery security materials associated with their HPLMN. This is also different from the existing 5G ProSe UE-to-Network Relay discovery security mechanism defined in TS 33.503 [1], where the Remote UE uses one from multiple sets of discovery security materials associated with the HPLMN of potential UE-to-Network Relay(s).

# 4	Detailed proposal

It is proposed that SA3 approved the below changes for inclusion in the draft TR [2].

**** START OF CHANGES ****

### 6.7.3	Evaluation

The solution fulfills the security requirements of Key Issue #1 for the multi-hop UE-to-network Relay discovery with Model A and Model B.

The UE-to-network Relay discovery procedure is reused for the multi-hop UE-to-network Relay discovery. By reusing the existing discovery mechanism, the Remote UE can securely discover the UE-to-network Relay via Intermediate Relay(s).

This solution ensures that the Intermediate Relay has been authenticated and established a connection to the network before serving the Remote UE (for discovery or connection), which is aligned with the SA2 conclusion.

In this solution, the Intermediate Relay needs to obtain the discovery security materials associated with its HPLMN and multiple sets of discovery security materials associated with the HPLMN of potential UE-to-network Relay(s)/Intermediate Relay(s)/Remote UE(s). Also, for discovery with Model B, a UE-to-Network Relay needs to obtain multiple sets of discovery security materials associated with the HPLMN of Intermediate Relay(s)/Remote UE(s). This deviates from the discovery security materials provisioning procedure for 5G ProSe UE-to-Network Relay defined in TS 33.503 [5].

In this solution, Remote UE, Intermediate Relay, and UE-to-Network Relay protect the discovery message using the discovery security materials associated with their respective HPLMN. This is different from the existing 5G ProSe UE-to-Network Relay discovery security mechanism defined in TS 33.503 [5], where the Remote UE uses one from multiple sets of discovery security materials associated with the HPLMN of potential UE-to-Network Relay(s).

Editor’s Note: Each solution should motivate how the potential security requirements of the key issues being addressed are fulfilled.

Editor’s Note: The need for Intermediate Relay to obtain the discovery security materials associated with its HPLMN and multiple sets of discovery security materials associated with the HPLMN of potential UE-to-network Relay(s)/Intermediate Relay(s) is FFS.

Editor’s Note: How the solution protects the path information during the discovery of multi-hop U2N relay is FFS.

**** END OF CHANGES ****