3GPP TSG-SA3 Meeting #118	S3-244159

Hyderabad, India  14 – 18 October 2024

# Source:	Ericsson

# Title:	Solution for accessing UE resources by an application on another UE

# Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

It is proposed to add the following solution to the TR 33.700-22.

# 2	References

[1]	3GPP TR 33.700-22: "Study on security aspects of CAPIF Phase3".

# 3	Rationale

This document introduces a solution to TR 33.700-22 [1] for key Issue #3 (Authorizing API invoker on one UE accessing resources related to another UE).

# 4	Detailed proposal

It is proposed to approve the following change to TR 33.700-22 [1].

***START OF 1st CHANGE***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

[XX]	3GPP TR 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

***START OF THE CHANGE***

### 6.X	Solution #X: Backend based solution for UE-deployed API invoker accessing resources not owned by that UE

#### 6.X.1 	Introduction

This solution addresses the requirements identified in key Issue #3 (Authorizing API invoker on one UE accessing resources related to another UE).

In the real word application deployments, it is very common practice to have a backend server in addition to the application clients running/installed on end point devices such as mobile phones. For example, when the use case of gaming application is considered, even if the game application client instances are running on different devices and communicating with each other directly, there is always a backend server serving these clients. Thus, utilization of backend server can bring some benefits in terms of handling access permission and user identifiers. Taking these benefits into account, this solution proposes that when an application client instance running on a UE wants to access resources related to another UE which hosts another application client instance, the requester client can send the request to the backend server and then the backend server can consume the related APIs and returns the result back to the requester client. From 3GPP point of view, only permission check whether the backend server as AF is allowed to access the resources of the UE is needed. The remaining permission check such as which users’ resource can be accessed by which users can be handled in the application layer out of 3GPP scope.

#### 6.X.2	Solution details

Figure 6.X.2-1 illustrates the high-level architecture.

Figure 6.X.2-1: High level architecture

High-level steps of the solution are presented in Figure 6.X.2-2 and explained below.

Figure 6.X.2-2: High-level procedure flow of the solution

1.	The application instance running on UE 1 sends a request to the backend server to access resources related to user 2. The communication between the application instance and backend server and its security are out of scope of the solution.

2.	The backend server identifies the UE ID of UE 2 related to the request. How to identify the UE ID is out of scope of the solution. Also, the backend server handles the resource owner authorization in the application layer (e.g., checks if user 1 can access resources related to user 2, gets permission from user 1). Application layer resource owner authorization handling is out of scope of the solution. The backend server acting the role of AF obtains an access token to access resources related to UE 2 by executing authentication and authorization the steps specified in TS 33.122 [XX] if there is no valid access token.

3.	The backend server acting the role of AF invokes the API of the AEF with the access token.

4.	After successful authentication of the backend server (i.e., AF) and verification of the access token, the AEF processes the API invocation and returns the result.

5.	The backend server can return the result.

#### 6.X.3	Evaluation

TBD.

***END OF THE CHANGE***