3GPP TSG-SA3 Meeting #118	S3-244161

Hyderabad, India  14 – 18 October 2024

# Source:	Ericsson

# Title:	Resolving ENs in solution #6

# Document for:	Approval

Agenda Item:	5.14

# 1	Decision/action requested

It is proposed to approve the following updates for solution #6 in TR 33.749.

# 2	References

[1]	3GPP TR 33.749: "Study on security aspects of enhancement of support for edge computing in the 5G Core (5GC) phase 3".

# 3	Rationale

This document resolves the ENs in solution #6 by providing more details.

# 4	Detailed proposal

It is proposed to approve the following change to TR 33.749 [1].

***START OF THE CHANGE***

## 6.6	Solution #6: UE ID token

### 6.6.1	Solution overview

This solution proposes usage of UE ID token to address the security requirements key issue #2.1. With the usage of UE ID token, there will be no need to share private IP address with EEC and other entities in the EC architecture.

### 6.6.2	Solution details

High-level overview of the solution is presented in figure 6.6.2-1 and steps are explained in detail below.

Figure 6.6.2-1: UE ID token based solution

Step 1:	The EEC obtains a UE ID token from the UE ID server which is located inside the operator domain. UE ID server can identify the UE by using the private IP address of the PDU session. For the UE ID Server service consumption by the EEC, the UE where the EEC is running on needs to execute the primary authentication successfully and have a PDU session.

Step 2:	The EEC sends the token to the EES to learn edge specific UE ID.

Step 3:	The EES invokes the NEF IP by sending the UE ID token.

Step 4:	The NEF interacts with the UE ID server to identify the UE.

Step 5:	The NEF sends the UE ID to the EES.

Step 6:	The EES sends the response to the EEC.

The UE ID Server is an AF function located inside the HPLMN trust domain and before the execution of NAT. In step 1, the UE ID server uses the UE IP address to identify the UE. For the execution of step 4, the NEF needs to be able to consume the UE ID Server service to obtain the UE ID by providing the UE ID token.

### 6.6.3	Solution evaluation

Editor’s note: Evaluation is FFS.

Editor’s note: The evaluation should state the applicability of the solution, i.e. what is the solution solving and what are the assumptions.

***END OF THE CHANGE***