3GPP TSG-SA3 Meeting #118	S3-244163

Hyderabad, India  14 – 18 October 2024

# Source:	Ericsson

# Title:	Security requirements for KI#1.1

# Document for:	Approval

Agenda Item:	5.14

# 1	Decision/action requested

It is proposed to approve the following security requirement for KI#1.1 in TR 33.749.

# 2	References

[1]	3GPP TR 33.749: "Study on security aspects of enhancement of support for edge computing in the 5G Core (5GC) phase 3".

# 3	Rationale

This document introduces potential secure requirement for KI#1.1 in TR 33.749 [1].

# 4	Detailed proposal

It is proposed to approve the following change to TR 33.749 [1].

***START OF THE CHANGE***

### 5.2.1	Key Issue #1.1: Security aspects related to enhancements of EAS and local UPF (re)selection.

#### 5.2.1.1	Key issue details

Several enhancements in the procedures for EAS and local UPF (re)selection have been proposed in TR 23.700-49 [2]. It has been concluded that N6 delay measurement is a factor of decision in those procedures. The N6 delay measurement between the termination points, i.e., UPF and EAS, is to be performed using existing IETF standard protocols (e.g., ICMP, TWAMP, OWAMP, etc.). The 5G network and the EDGE negotiate and exchange information about the termination points, protocols and parameters to be used, and then the SMF (“controller”) configures the UPF(s) to conduct the measurements as “senders” to the network devices/compute nodes/EAS(s) acting as “reflectors/receiver”. The UPF(s) will report the results to SMF.

The IETF RFCs corresponding to the IP Performanace Metrics (IPPM) defined protocols, such as RFC 4656, RFC 5357, RFC 8762, etc., include security considerations to be taken into account when those protocols are in use. RFC 2681 [yy] refers to two types of security concerns in these metrics: the potential harm caused by measurements, and the potential harm to the measurements.

The security risks associated with each protocol need to be tackled in the context of EAS and local UPF (re)selection, assuming that EDGE servers, such as the EAS, are placed outside the operator domain.

#### 5.2.1.2	Security threats

Denial of Service: Potential attackers can exploit the capabilities of protocols like ICMP to overwhelm targeted servers like EAS (e.g., ping flood, smurf attacks, etc.).

Spoofing and Theft of service: Without authentication between, the termination points, the client may fake the source address of other device and use the measurement services produced by the server/reflector/receiver.

Tampering: The measurement traffic can be compromised by for example a man-in-the-middle entity, which can inject fake measurement traffic, or give to that a different priority, etc. Consequently, the selection of the server (UPF or EAS) could be biased by the attacker to their interest, leading in the worst-case scenario to the selection of a UPF or EAS already compromised and/or under their control.

#### 5.2.1.3	Potential security requirements

For the secure N6 delay measurement the UPF and the EAS should support mutual authentication and integrity protection mechanism.

***END OF THE CHANGE***