3GPP TSG-SA3 Meeting #118	S3-244167

Hyderabad, India  14 – 18 October 2024

# Source:	Ericsson

# Title:	Update to KI#1.2

# Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

It is proposed to add the following key issue update to the TR 33.700-22.

# 2	References

[1]	3GPP TR 33.700-22: "Study on security aspects of CAPIF Phase3".

# 3	Rationale

This proposes a new requirement to be added in key issue #1.2 (Resource owner authorization management) of TR 33.700-22 [1].

# 4	Detailed proposal

It is proposed to approve the following change to TR 33.700-22 [1].

***START OF THE CHANGE***

### 5.1.2	Key Issue #1.2: Resource owner authorization management

#### 5.1.2.1	Key issue details

KI#1 of TR 23.700-22 [3] is studying resource owner authorization management (e.g., authorizing access to the resource owner's resource or revoking the authorization of access to the resource owner's resource). There is a NOTE in TR 23.700-22 [3]:

NOTE:	Aspects pertaining to the definition of resource owner consent/authorization over CAPIF-8 are in the scope of SA3, noting that the R18 security aspects of CAPIF supporting RNAA are specified in 3GPP TS 33.122 [3].

This key issue studies how to authenticate and authorize the resource owner to provide resource owner authorization.

As from clause 6.5.3. in TS 33.122 [3], (for Release 18) three authorization flows are introduced to be used as potential procedures in RNAA enhanced CAPIF. These are client credentials, authorization code and authorization code with PKCE. Client credentials flow generally becomes applicable when the authorization is preconfigured in the authorization server by an out of band mechanism. The authorization code and authorization code with PKCE require online user interaction. The case where there is no preconfigured authorization information and the authorization from the user can only be obtained by an out of band mechanism is not addressed. New authorization flow(s) need to be studied to address that case.

#### 5.1.2.2	Security threats

Without the authentication between resource owner and authorization server, malicious resource owner can impersonate victim resource owner to do resource owner authorization management.

#### 5.1.2.3	Potential Security Requirement

Mutual authentication between the authorization server and the resource owner should be supported.

CAPIF RNAA should support to authorize the resource owner to provide resource owner authorization.

CAPIF RNAA should support authorization of API invoker based on resource owner authorization and should support revocation of the resource owner authorization.

CAPIF RNAA should support an authorization flow also addressing the case that the resource owner is not available online and there is no resource owner permission obtained in advance, for service availability.

***END OF THE CHANGE***