3GPP TSG-SA3 Meeting #118	S3-244201

Hyderabad, India  14 – 18 October 2024

Title:	Draft_Reply-LS on PQC Migration

Response to:	LS PQTN#70 Doc_003 on LS reply to 3GPP Reply-LS on PQC Migration from GSMA

Release:

Work Item:

Source:	Ericsson (to be SA WG3)

To:	GSMA PQTN

Cc:

Contact person:	Mohsin Khan

mohsin.a.khan@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

SA3 thanks GSMA for the LS on migration to Post-Quantum Cryptography (PQC). SA3 would like to provide the following responses to the raised questions:

- Q1: Regarding “SA3 considers that all the 128-bit algorithms currently used in 5G systems can be considered as secure against attacks from both classical and quantum computing.”, As it is known, the NIST was endorsed the AES-128, quoted “remain secure for decades” [2]. For SNOW-128 and ZUC-128 algorithms, is there any endorsement or analysis that SA3 could provide as the reference to prove these algorithms can be considered quantum-safe?

[SA3]: A very good summary of the impact of Cryptographically Relevant Quantum Computers (CRQCs) on symmetric cryptography was recently given in a statement by the Internet Engineering Task Force (IETF) [1]. SA3 agrees with IETF’s analysis. AES-128 can be implemented with a relatively small number of gates. Most other 128-bit algorithms such as SNOW 3G, ZUC, TUAK, KMAC128, Ascon, etc. are likely to require more gates. Even if an algorithm requires slightly fewer gates than AES-128, it still fulfills the requirement (comparable to key search on AES-128) for quantum resistance category 1 [2]. As stated by UK CNSA: “symmetric algorithms with at least 128-bit keys (such as AES) can continue to be used” [3]. The exact inner detail of the symmetric algorithm is not significant to the running time of Grover’s algorithm.

Is there any analysis that GSMA could provide to prove that SNOW 3G and ZUC are not quantum-safe? As estimated by the distinguished Professor Scott Aaronson, Schlumberger Centennial Chair of Computer Science, and Director of the Quantum Information Center at The University of Texas, over 95% of all claims about speedups over classical computers are incorrect [4]. Biased work or claims designed to increase interest or urgency are often presented by companies that sell quantum computing products, consulting services, discovery tools, or proprietary security solutions.

- Q2: Regarding “3GPP profiles of these protocol”, What exact the 3GPP profiles are? Could SA3 provide some guidance on the 3GPP profiles to prioritize the protocols to be migrated/mandatory.

[SA3]: The 3GPP profiles for IKEv2, TLS, DTLS, JWS, X.509, CRL, OCSP, CMP and SUPI encryption can be found in 3GPP TS 33.210 [5], 3GPP TS 33.310 [6], and 3GPP TS 33.501 [7]. 3GPP is planning to update all profiles using asymmetric algorithms with PQC. 3GPP has not discussed if the new PQC algorithms will be introduced as may/should/shall support/use. The order of migration is dependent on IETF work on introduction of PQC algorithms in protocol specifications. The status of IETF’s work on PQC migration is summarized in [1].

## References:

[1] IETF Statement on Quantum Safe Cryptographic Protocol Inventory
https://datatracker.ietf.org/liaison/1942/

[2] NIST FIPS 203 (Initial Public Draft)
https://nvlpubs.nist.gov/nistpubs/FIPS/NIST.FIPS.203.ipd.pdf

[3] UK NCSC Next steps in preparing for post-quantum cryptography
https://www.ncsc.gov.uk/whitepaper/next-steps-preparing-for-post-quantum-cryptography

[4] Aaronson, Quantum Computational Supremacy
https://www.youtube.com/watch?v=QnLmGfNKCLU&t=2710s

[5] 3GPP TS 33.210 Network Domain Security (NDS); IP network layer security
https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=2279

[6] 3GPP TS 33.310 Network Domain Security (NDS); Authentication Framework (AF)
https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=2293

[7] 3GPP TS 33.501 Security architecture and procedures for 5G System
https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=3169

# 2	Actions

To GSMA PQTN

ACTION: 	SA3 kindly asks GSMA to take above information into account.

# 3	Dates of next TSG SA WG 3 meetings

SA3#119	11 - 15 November 2024		Orlando (US)

SA3#120	17 – 21 February 2025		Athens (Greece)