3GPP TSG-SA3 Meeting #118	S3-244251

Hyderabad, India  14 – 18 October 2024	(revision of xx-yyxxxx)

# Source:	Motorola Mobility [Rapporteur]

# Title:	New WID on enablers for Zero Trust Security

# Document for:	Discussion

# Agenda Item:	5.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: New WID on enablers for Zero Trust Security

Acronym: eZTS_SBA

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-19

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
| X? | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_ZTS | SA3 | 960038 | Study on Zero Trust Security principles in mobile networks |
| FS_eZTS | SA3 | 1020034 | Study on enablers for Zero Trust Security |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification: NA

# 3	Justification

Release-18 TR 33.894 [1] performed analysis of 5G core SBA security aspects and identified that authentication, authorization, and communication security were supported between NF service consumers and NF service producers by mean of standard mechanisms specified in TS 33.501. But there is no standardization support available for data collection to enable operator to perform security evaluation and monitoring to identify potential NF compromise in advance in order to improve access control security at the SBA layer to prevent further threat lateral movement. Further [1] clause 5.1.5.3 specifically for the data collection and exposure aspect, it states that, ‘with not yet specified data collection, this information needs to be well defined and explicitly specified to allow for interoperability and secure operation of installed base’. Further Release-19 TR 33.794 [2] performed the study to identify the potential security scenarios where data collection and exposure can help to identify the threats/attacks on 5G SBA layer. Specifically for the related KI#1 ‘Data exposure for security evaluation and monitoring’, the conclusion identifies a list of security incidents or scenarios in SBA where data can be collected in the SBA layer and further details can be referred in TR 33.794 conclusion.

Further TR 33.794 study investigated the potential security policy enforcement use case for SBA access control decision enhancement and for the related KI#2 Security mechanisms for policy enforcement at the 5G SBA following aspects were concluded:

[TBD]

Editor’s Note: KI#2 conclusion aspects are FFS.

References:

[1] 3GPP TR 33.894, "Study on applicability of the zero trust security principles in mobile networks", Release 18.

[2] 3GPP TR 33.794: "Study on enablers for Zero Trust Security", Release 19.

# 4	Objective

The objectives of this work item include the following aspects according to the conclusions from TR 33.794:

- Define the system security requirements for the following security incidents or scenarios to support security event(s) data collection at SBA layer and exposure to Operator’s Security Function to enable security evaluation and monitoring. Further related details and if there need be a mandatory/optional support is upto discussions in the normative phase.

1)	authentication and authorization failure event;

2)	unexpected setup of TLS session and API invocation related to unauthorized reconnaissance;

3)	malformed message event;

4)	high service load;

5)	unexpected SBI call flows; and

6)	unexpected use of APIs exposed by services in SBA layer.

- [High-level conclusion text from KI#2].

Editor’s Note: Further details are FFS.

# 5	Expected Output and Time scale

[This part is to be discussed further]

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| External TR? |  |  |  |  |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 33.501? | Security architecture and procedures for 5G system | SA#108 (June 2025) | {Free text, e.g. "This TS covers Stage 2" or This TS covers both stages 2 and 3"} |

# 6	Work item Rapporteur(s)

# 7	Work item leadership

Primary: SA3

Secondary: TBD (SA2, SA5, CT WGs?)

# 8	Aspects that involve other WGs

TBD

The “aspects” can be provided by topic (e.g. “security”, “multimedia”) and/or by specifying the WG(s) e.g.: "SA2, SA3, SA5, SA6. CT6 for storage, and potentially SA4". If not applicable, indicate "None" or "None identified yet"}

For a Stage 2 WID requiring Stage 3 to be done by another group: on a best-effort basis, indicate which potential WG is expected to specify the Stage 3: {possible values: "Not applicable", " unknown", "CT WGs", etc}

# 9	Supporting Individual Members

Supporting IM name