3GPP TSG-SA3 Meeting #118	S3-244279

Hyderabad, India 14 - 18 October 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	Solution for KI#3 on Security aspects of digital asset container in 5G

Document for:	Approval

Agenda Item:	5.18

# 1	Decision/action requested

Approve the solution added to TR 33.721

# 2	References

[1]	3GPP TR 23.700-21 Study on Application enablement architecture for mobile metaverse services

[2]	3GPP TR 33.721 Study on security aspects of 5G Mobile Metaverse services

[3]	3GPP TS 33.501 Security architecture and procedures for 5G system

# 3	Rationale

Avatar and digital asset support, including digital asset avatar management and discovery, is discussed in Key issue #3 in TR 23.700-21, which requires to create, update, retrieve, delete and discover digital assets securely.

Correspondingly, KI#3 on Security aspects of digital asset container in 5G was approved and added in TR 33.721.

This solution aims to address the requirement of the KI to authenticate and authorize a digital asset service consumer to access the digital asset(s) in a digital asset container.

# 4	Detailed proposals

All content is new.

# * * * * First change* * * *

## 6.x	Solution #x: authenticate and authorize digital asset service consumer to access digital asset in DAC

### 6.x.1	Introduction

This solution addresses Key Issue #3 on Security aspects of digital asset container in 5G.

Avatar and digital asset support, including digital asset avatar management and discovery, is discussed in Key issue #3 in TR 23.700-21[2]. The corresponding requirements were defined in clause 5.4 of the TR 23.700-21[2] which include, for example, subject to operator policy, regulatory requirements and user consent, the metaverse enablement service shall provide digital asset management mechanisms as follows:

-	to create, update, retrieve, delete and discover digital assets securely.

-	to manage associations between digital assets and user identifiers.

-	to allow an authorized third party to manage digital asset(s) associated with a user.

Correspondingly, KI#3 on Security aspects of digital asset container in 5G was described in TR 33.721 which requires 5G system to authenticate and authorize a digital asset service consumer to access the digital asset(s) in a digital asset container.

This solution proposes TLS for mutual authentication between digital asset service consumer and digital asset container, and UE token for authorization of digital asset service consumer on behalf of the UE to access digital assets associated with the UE .

### 6.x.2	Solution details

Figure 6.x.2-1 Procedure to authorize digital asset service consumer by DAC

Assumption:

UE may play roles of VAL client, SEAL client or digital asset service (DAS) consumer.

Digital Asset Container (DAC) may play roles of SEAL server or DAS producer.

AF may play roles of VAL server, SEAL client or DAS consumer.

TLS based mutual authentication is performed between UE and AF, AF and DAC. UE and DAC.

Certificate of UE to sign UE token is preconfigured in AF and DAC.

1. UE generates an authorization token consisting of AF_ID, UE id, asset type, asset id, expiry, usage count and signature signed with UE's certificate.

2. If UE needs digital asset to support application client, UE sends request to DAC which includes UE id, asset type, asset id and authorization token generated in step 1.

3. DAC verifies signature, expire time and usage count of the token, and check if the required digital asset is associated with the UE.

4. If verification and checking are passed, DAC returns digital asset to the UE.

5. If metaverse service provided by an application server need digital asset associated with the UE, UE sends metaverse service request to the AF which includes UE id, asset type, asset id and authorization token generated in step 1 together with other information.

6. AF verifies signature, expire time and usage count of the token. If UE authorization token is not included in step 5, AF may send request to UE for the authorization token.

7. If verification is passed, AF sends request to DAC which includes UE id, asset type, asset id and authorization token received in step 5.

8. DAC verifies signature, expire time and usage count of the token, and check if the required digital asset is associated with the UE and AF ID in the token is matched the AF ID of the requesting AF.

9. If verification and checking are passed, DAC returns digital asset to the AF.

10. The AF continue metaverse service for the UE with the digital asset associated with the UE.

### 6.x.3	Evaluation

TBA.

# * * * * End of changes * * * *