3GPP TSG-SA3 Meeting #118	S3-244390

Hyderabad, India 14 - 18 October 2024												revision of S3-244133

# Source:	Qualcomm Incorporated

# Title:	Clarifications on the IOPS enhancements in solution #3

# Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

This contribution provides some clarifications to the IOPS enhancement in solution #3.

# 2	References

[1]	S3-243555 Draft TR 33.700-29 v0.4.0

# 3	Rationale

There are still two Editor’s Notes in solution #3 and this contribution proposes changes to address them. Some text in the evaluation is also moved to make the text read better.

# 4	Detailed proposal

It is proposed to include the below pCR in the draft TR [1].

**** START OF CHANGES ****

## 6.3	Solution #3: IOPS based solution for UE to satellite security

### 6.3.1	Introduction

This solution addresses key issues #1 and #2.

It applies to an architecture when a complete network is deployed in the satellite or set of interlinked satellites.

### 6.3.2	Solution details

#### 6.3.2.1	IOPS based solution

This solution applies to the case that the whole network, i.e. serving and home network, is hosted in the satellite or set of satellites. For EPS this includes deploying an HSS in the (set of) satellite(s). In this case all the security procedures that are used between the UE and satellite(s) are the same one as used between the UE and network in regular 3GPP access.

Such a type of solution requires the pre-configuration of credentials that are used to authenticate with the UE in the satellite. In order to enable different keys to be configured in different satellites for the same UE, it is proposed to use a solution like the one described in Annex F.4 of TS 33.401 [3].

NOTE: As all the parameters used in Annex F.4 of TS 33.401 [3] relate to the authentication between the UE and network and are in effect under an operator’s control, solution similar to one described could be used.

The solution described in Annex F.4 of TS 33.401 [3] uses bits of the AMF field in the AUTN parameter and also possibly the IND part of the sequence number SQN, as described in Annex C.1 of TS 33.102 [7] to calculate a root key for the authentication between the UE and particular (set of) satellite(s). This means that a different key can be used between the UE and each different (set of) satellite(s). This is achieved using existing information and hence requires no update to the signalling that is used between UE and regular networks.

The solution supports roaming by having the relevant key for the UE provisioned into the satellite PMLN’s HSS (which does not have to be the UE’s HPLMN). Provisioning such a key enables the UE to access the PLMN supported by the satellite.

With this solution, there is only a need to re-establish the security when the UE moves away from the set of interlinked satellites and the overall processing in the HSS in authenticating a UE is the same regardless of the location of the HSS. The authentication latency is small and the reliability is high with the HSS in the satellite(s) as it is always possible to authenticate the UE in a single uninterrupted operation.

#### 6.3.2.2	Enhancement to IOPS solution

One part of the IOPS solution that is not discussed in the above clause is the way a new key can be calculated for a particular value of the AMF field in the AUTN parameter by using a value m for each value of the AMF field. An example of the use of value m is described in Annex F.4.2 of TS 33.401 [3]. The effect of that Annex is to base the key derivation on the m value in addition to the n value and hence old keys are in effect deprecated.

The following describes a method that could be used to automatically update the value of m. This is done by including m in the SQN number that is sent to the USIM, e.g.

SQN = (45 bit) m value | (3 bit) x value

The least significant bits of m would need to be sent in the clear, i.e. not encrypted by AK, so that the USIM can get estimate the value of m.  In this example, the HSS in the satellite would only be able to generate at most 8 (=23) AVs because change of m value requires a new key to generate AV. This illustrates how this can be used to limit the effect of a compromise of onboard HSS in the satellite and of course different lengths of m can be chosen in practise. The USIM would store the largest m value used for successful authentication for a particular n value and not accept any AVs with  an m value that is less than the stored m value. When the HPLMN provides a new key to the satellite, the HPLMN also provides the value of m to use with that key. This enables the satellite to create the SQN for the AV embedding the value of m to be used by the USIM to ensure that the HSS in the satellite and USIM use the same value of m when creating or processing the AV respectively.

Editor’s note: Further details on the enhancement are needed.

The result of this auto-updating of the m values is that an old key gets deprecated after the successful reception of an AV with a larger  m value in the SQN. So for example if a HSS in a satellite always is given a fixed value of n for a particular UE, then once an AV has been processed by the USIM with a particular value of m, then all smaller values of m will not result in a successful processing of the AV by the USIM. This deprecation happens in-band during AV processing rather than by out-of-band methods as currently expected in Annex F.4.2 of TS 33.401 [3]. This is a  similar security property to that of AVs generated in a HPLMN HSS in regular deployments.

### 6.3.3	Evaluation

The solutions described in clauses 6.3.2.1 and 6.3.2.2 are applicable to the case that the full core network is on board the satellite.

The solutions rely on IOPS solution described in TS 33.401 [3] and does not require further changes to the LTE security procedure other than the IOPS related ones.

It is left out of scope of the solution how the HSSs on the satellites are provisioned with the keys etc.

The enhancement described in clause 6.3.2.2, enables the automatic deprecation of the key(s) deployed in the HSS(s) deployed in satellites, relies on deployment of USIM supporting IOPS with the enhancement described in TS 33.401 [3]. The HSSs in satellite will also need to support generation of AVs that is compatible with such USIMs. This involves a slightly modified handling of the SQN numbers used in AVs compared to regular 3GPP accesses and calculation of keys compared to the existing IOPS solution.

The enhancement of the IOPS solution given in clause 6.3.2.2 deprecates the keys in HSS in a similar manner that old AVs are deprecated in regular LTE.

Editor’s Note: The impacts of the key lifetime limitation proposed in clause 6.3.2.2 are FFS.

**** END OF CHANGES ****