3GPP TSG-SA3 Meeting #118	Draft_S3-244429r2

erabad, India  14 - 18 October 2024

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Enhancements in solution 3 for KI#2.1

# Document for:	Approval

Agenda Item:	5.14

# 1	Decision/action requested

It is required to approve the enhancements proposed in this contribution for solution 3 of the study

# 2	References

[1]	3GPP TR 33.749 v0.3.0 “Study on security aspects of enhancement of support for edge computing in the 5G Core (5GC) phase 3”

# 3	Rationale

This contribution is intended to clarify some aspects of the solution #3 of [1], as well as to resolve the editor’s notes with the following rationale:

Editor’s Note: the details on which NF generates the temporary ID is ffs.

The Temporary ID is generated in the 5G Core Network. It is proposed that UDM is the NF generating that, but eventually the SMF might be a candidate for that, as it is related with the PDU session.

Editor’s Note: the details of the temporary ID are ffs.

The Temporary ID consists of a random part and an identifier of the NF (e.g., UDM) that generated it.

Editor’s Note: how the EEC is authorized to get the assigned temp ID is ffs.

EEC is indeed the entity requesting the temporary ID to the UE, so it is implicitly allowed to use it once returned from the 5G Core.

Editor’s Note: how the NEF fetches the temporary ID from the NF which generates it to make the verification is ffs.

The NEF will use the UDM/UDR information contained in the Temporary ID to contact the correct UDM/UDR and retrieve the corresponding UE Id.

Editor’s Note: The evaluation should state the applicability of the solution, i.e. what is the solution solving and what are the assumptions.

Additional text has been added indicating that the solution applies to both security requirements indicated in KI#2.1, as well as the impact in the overall architecture, i.e., basically the generation of the new temporary identifier.

Editor’s Note: Further evaluation is to be defined.

No further evaluation points have been identified.

# 4	Detailed proposal

## 6.3	Solution #3: Secure retrieval of 5G UE Id and privacy related information

### 6.3.1	Introduction

This solution addresses the security requirements exposed in key issue #1 related to the secure retrieval of 5G system UE Id and privacy related information, i.e., the verification of the user information provided by the EEC, and the protection of the UE privacy.

The main design principle of the solution is based on the utilization of a temporary identifier (ID) as UE identity to fetch the Application Function (AF) specific UE Id from Nnef_UEId service. The temporary ID is generated by the 5G Core (e.g., UDM), it includes a randomly generated value, and it is associated to the PDU session (eventually to the AF ID).

### 6.3.2	Solution details

Figure 6.Y.2-1 illustrates the procedure to fetch the 5G system UE Id and privacy related information.

Figure 6.Y.2-1: Procedure to fetch the 5G system UE Id and privacy related information

Precondition:

Step 0. The EEC requests a Temporary ID to UE through interface provided by UE module.

1. PDU session establishment / modification request is sent from the UE to the 5G system.

2. The 5G Core (e.g., UDM) generates (per UE) an internal temporary ID (with a randomly generated part) for PDU session.

NOTE 1: The UE could request multiple temporary IDs during the same PDU session (PDU Session modification). The UE can use this to invalidate previous IDs which might have been compromised and/or at fix intervals without initializing another PDU session establishment/modification.

NOTE 2: The Temporary ID consists of a random part and an identifier of the UDM (UDM group ID) that generated the Temporary ID.

NOTE 3: The Temporary ID could also be generated from SMF and sent to UDM/UDR for storage.

3. The 5G Core returns the internal temporary ID to the UE as part of the NAS PDU session establishment/modification response.

4. The EEC uses the allocated temporary ID, received from the UE, inside the Edge application layer communication with the application function AF (e.g., EAS).

5. The AF uses the received temporary ID to invoke the Nnef_UEId API intended to fetch the 5G system UE identifier, i.e., AF specific UE Id, as per the procedure described in TS 23.502 figure 4.15.10-1 [8].

6. The NEF verifies the temporary ID, authorizes the request coming from the AF and accordingly retrieves from the 5G Core the AF specific UE Id corresponding to the temporary ID.

NOTE 4: NEF retrieves the correct UDM instance from the UDM identifier in the Temporary ID, and NEF queries UDM to obtain the UE ID from the temporary ID.

7. The NEF replies to the AF with the corresponding AF specific UE Id.

8-9 (Optional). Assuming the application logic requires to fetch the UE location, a location request is sent to the NEF using the AF specific UE Id, and the corresponding information is provided by the 5G system through the NEF.

10. The AF responds to the UE (EEC).

### 6.3.3	Evaluation

The solution requires the generation of a new temporary ID identifier by the 5G Core, e.g. UDM or SMF, that needs to be used in the invocation of Nnef_UEId API services.

The solution requires UE module to support Temporary ID request from application layer.

The solution avoids the mapping of potentially long-term and easily inferred identifiers, with the 5GS UE Id, to block malicious applications from performing UE tracking type of attacks.

The solution solves the issue of verification of the user information provided by the EEC through the generation of a temporary ID in 5G Core per UE and PDU session, that is verified in the NEF when Nnef_UEId or other APIs (e.g., location) are invoked by the AF(s). The solution also solves the potential privacy concerns during the retrieval process of 5G system UE Id by avoiding the use of permanent or semi-permanent identifiers.

Editor’s Note: Further evaluation is to be defined.