3GPP TSG-SA3 Meeting #118	S3-244431

Hyderabad, India  14 – 18 October 2024	revision of S3-244162

# Source:	Ericsson

# Title:	Evaluation of solution #6

# Document for:	Approval

Agenda Item:	5.14

# 1	Decision/action requested

It is proposed to approve the following evaluation for solution #6 in TR 33.749.

# 2	References

[1]	3GPP TR 33.749: "Study on security aspects of enhancement of support for edge computing in the 5G Core (5GC) phase 3".

# 3	Rationale

This document evaluates solution #6 in TR 33.749 [1] by providing solution applicability and impact analysis.

# 4	Detailed proposal

It is proposed to approve the following change to TR 33.749 [1].

***START OF THE CHANGE***

## Solution #6: UE ID token

### 6.6.1	Solution overview

This solution proposes usage of UE ID token to address the security requirements key issue #2.1. With the usage of UE ID token, there will be no need to share private IP address with EEC and other entities in the EC architecture.

### 6.6.2	Solution details

High-level overview of the solution is presented in figure 6.6.2-1 and steps are explained in detail below.

Figure 6.6.2-1: UE ID token based solution

Step 1:	The EEC obtains a UE ID token from the UE ID server which is located inside the operator domain. UE ID server can identify the UE by using the private IP address of the PDU session.

Editor’s note: Further details, such as how the EEC authenticates with the UE ID server of the operator, are FFS.

Step 2:	The EEC sends the token to the EES to learn edge specific UE ID.

Step 3:	The EES invokes the NEF IP by sending the UE ID token.

Step 4:	The NEF interacts with the UE ID server to identify the UE.

Step 5:	The NEF sends the UE ID to the EES.

Step 6:		The EES sends the response to the EEC.

Editor’s note: Clarification on which architecture the solution is based on is FFS.

### 6.6.3	Solution evaluation

Solution applicability evaluation:

- The solution addresses both the privacy and prevention against spoofing attacks requirements of key issue #2.1.

- The solution works also for the case that the NAT operation is performed after the UPF.

Solution impact evaluation:

- The procedure in TS 23.558 clause 8.6.5.2 needs minor update to allow the EEC to use the UE ID token instead of UE IP address.

- A new AF (UE ID Server) needs to be introduced.

- A new interface between the UE ID Server and the NEF for the NEF to be able to consume the UE ID Server service is required.

- EES Eees_UEIdentifier API and NEF's Nnef_UEId API needs minor update to allow the use of UE ID token as an input parameter.

Editor’s Note: Further evaluation is FFS.

***END OF THE CHANGE***