3GPP TSG-SA3 Meeting #118	Draft-S3-244437r5

Hyderabad, India  14 – 18 October 2024

# Source:	Nokia, Nokia Shanghai Bell, Ericsson

# Title:	Evaluation and conclusion of KI#1.1

# Document for:	Approval

Agenda Item:	5.14

# 1	Decision/action requested

It is requested to approve this contribution as evaluation and conclusion of KI#1.1.

# 2	References

[1]		3GPP TR 33.749 v0.3.0 “Study on security aspects of enhancement of support for edge computing in the 5G Core (5GC) phase 3”

[2]		3GPP TR 23.700-49: "Study on Enhancement of support for Edge Computing in 5G Core network - Phase 3".

# 3	Rationale

Based on the rationale and explanation provided in the discussion paper (S3-244281), the contribution is intended to evaluate the most relevant IETF protocols that can be used in N6 delay measurement from a security perspective, and provides a recommendation to move forward the feature in a secure manner.

It is also proposed to remove the editor’s note in the potential security requirements by a NOTE explaining that the evaluation of the security mechanisms has been added in the conclusion of the KI, clause 7.X.

Editor’s Note: The security mechanisms of the protocols to be used in N6 delay measurement are ffs.

# 4	Detailed proposal

*****************START of FIRST CHANGES*************************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-49: "Study on Enhancement of support for Edge Computing in 5G Core network - Phase 3".

[3]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[4]	3GPP TS 33.558: "Security aspects of enhancement of support for enabling edge applications; Stage 2".

[5]	3GPP TR 33.839: "Study on security aspects of enhancement of support for edge computing in the 5G Core ".

[6]	3GPP TR 33.739: "Study on security enhancement of support for edge computing phase 2".

[7]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[8]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[9]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)"

[W]	IETF RFC 4656: “A One-way Active Measurement Protocol (OWAMP)”

[X]	IETF RFC 5357: “A Two-Way Active Measurement Protocol (TWAMP)”

[Y]	IETF RFC 8762: “Simple Two-Way Active Measurement Protocol”

[Z]	IETF RFC 792: “INTERNET CONTROL MESSAGE PROTOCOL”

*****************END of FIRST CHANGES********************************************

*****************START of SECOND CHANGES**************************************

*****************END of SECOND CHANGES********************************************

*****************START of THIRD CHANGES**************************************

# 7	Conclusions

## 7.X	KI#1.1: Security aspects related to enhancements of EAS and local UPF (re)selection.

### 7.X.1	Evaluation

The security capabilities of the following IETF protocols for IP Performance Mettrics (IPPM) have been analyzed with the aim to evaluate their adequacy to be used to collect N6 delay measurements between local UPF and EAS:

- IETF RFC 4656: “A One-way Active Measurement Protocol (OWAMP)” [W]

- IETF RFC 5357: “A Two-Way Active Measurement Protocol (TWAMP)” [X]

- IETF RFC 8762: “Simple Two-Way Active Measurement Protocol” [Y]

- IETF RFC 792: “INTERNET CONTROL MESSAGE PROTOCOL” [Z]

The table 7.X.1-1 summaries the security capabilities specified per protocol:

Table 7.X.1-1: Summary of security capabilities of IPPM defined protocols

In conclusion of KI#2 of TR 23.700-49 (see 8.2 of [2]):

-	N6 delay between 5GC N6 termination point (i.e. UPF) and the measurement endpoint at application side (i.e. EAS IP address(es)/Designated IP (range)) is measured by leveraging existing mechanisms (e.g. ICMP, TWAMP, OWAMP or other protocols defined by IETF).

The following guidelines from the security point of view for the N6 delay measurement between UPF PSA and EAS will be documented:

- It is recommended to use of the measurement protocols supporting authentication, integrity, and anti-replay protection.

- It is recommended not to use a delay measurement mechanism that requires sensitive information exchange but does not support confidentiality by the measurement protocols.

- It is not recommended to use a measurement protocol without authentication, integrity and anti-replay protection in place.

- It is not recommended to use a measurement protocol with disclosed vulnerabilities which may be exploited for attack or blocked by the operators.

*****************END of THIRD CHANGES********************************************

*****************START of FOURTH CHANGES**************************************

*****************END of FOURTH CHANGES********************************************