3GPP TSG-SA3 Meeting #119AdHoc-e	S3-250106

Online, Electronic meeting, 13 -16 January 2025

# Source:	Ericsson

# Title:	Conclusion for key issue #3

# Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

It is proposed to add the following update to the TR 33.700-22.

# 2	References

[1]	3GPP TR 33.700-22: "Study on security aspects of CAPIF Phase3".

# 3	Rationale

A conclusion, based on the solution introduced in S3-250096, is proposed for the key issue #3 in TR 33.700-22 [1].

# 4	Detailed proposal

It is proposed to approve the following change to TR 33.700-22 [1].

***START OF THE CHANGE***

## 7.X	Conclusion for KI #3: Authorizing API invoker on one UE accessing resources related to another UE

The following principles will be followed in the normative work.

-	The API invoker residing on one UE can directly or via its backend server can invoke the NB API.

-	The Application identifier of the API invoker is used in the authorization decision.

-	Resource owner preference (e.g., an indication and policy) for allowing the CCF / Authorization Function / Resource owner management function to initiate the interaction with the ROF of the resource owner is checked before sending notification to the ROF of the resource owner.

-	Before interacting with the ROF of the target UE, the CCF checks the indication of allowing ROF usage for obtaining resource owner authorization and authorization revocation.

-	CIBA flow usage will be specified.

***END OF THE CHANGE***