3GPP TSG-SA3 Meeting #119AdHoc-e	S3-250116

# Online, Electronic meeting, 13 -16 January 2025

Source:	Xiaomi

Title:	Pseudo-CR on TR 33.700-22 KI#1.1 conclusion update

Document for:	Approval

Agenda item:	5.19

Spec:	3GPP TR 33.700-22

Version:		0.3.0

Work Item:	FS_CAPIF_Ph3-sec

Comments

The ROF can be the browser on the UE. Therefore, CCF’s root certificate can be embedded in to the ROF and the embedding method can be left to implementation. Then the ROF can obtain the CCF’s certificate via service request and check the CCF’s certificate via the CCF’s root certificate.  Therefore, the following EN is resolved.

Editor’s Note: It is ffs whether a mechanism on how ROF gets the CCF certificate needs to be specified.

CAPIF-8 is used to secure the delivery of resource owner’s authorization information or authorization revocation information. Before sending the information, the resource owner (i.e., the human) can be authenticated by the CCF via application layer (e.g., the resource owner can be authenticated by the password), thus the authentication of ROF is not needed. Hence, the following ENs are resolved.

Editor’s Note: The conclusion for authentication of the ROF is ffs.

Proposed Changes

* * * First Change * * * *

7.1.1	Conclusions for KI#1.1 CAPIF-8 reference point

Normative work is recommended to protect the CAPIF-8 reference point based on the following principles:

The authentication of the CCF is based on the CCF’s certificate.

The TLS secure channel is used to provide messages exchanged between the ROF and the CCF with integrity protection, confidentiality protection and. replay protection.

Editor’s Note: Further conclusions are ffs.

* * * End of Change * * * *