3GPP TSG-SA3 Meeting #119AdHoc-e	S3-250127

Online, Electronic meeting, 13 -16 January 2025

# Source:	Philips International B.V.

# Title:	KI#5 conclusions

# Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

SA3 is kindly requested to approve the following contribution.

# 2	References

# 3	Rationale

This contribution proposes conclusions for KI#5: Authentication in Ambient IoT service.

The AIoT device may be owned/managed by the network or a 3rd party, and as both parties may require authenticating the AIoT device(s), the means to achieve authentication on network-layer and application-layer need to be captured.

For application-layer based authentication, the procedures are outside the scope of 3GPP. However, the means to transport the data between the AIoT device and the Application Function (AF) need to be specified.

For network-layer based authentication, different use cases may have different authentication requirements, hence the need to support both one-way and mutual authentication.

# 4	Detailed proposal

*** START OF CHANGES ***

# 7	Conclusions

Editor’s Note: This clause contains the agreed conclusions that will form the basis for any normative work.

## 7.X	Key Issue #5: Authentication in Ambient IoT service

The following principles are agreed for the authentication in Ambient IoT service:

For network-layer based authentication:

- To provide Ambient IoT services and protect the network, Ambient IoT device(s) authentication is mandatory.

- Network authentication by the Ambient IoT device is mandatory to support, optional to perform. Whether the Ambient IoT device authenticates the network depends on the use case (e.g., to perform disable operation, the AIoT device needs to authenticate the network).

- Whether authentication is one-way or mutual, the authentication procedure is initiated by the network with the paging message carrying an authentication challenge.

NOTE: One-way and mutual authentication procedures’ details are to be specified during the normative phase.

For application-layer based authentication:

- Authentication data between Ambient IoT device and the application is carried in authentication containers whose contents are transparent to the network layer.

- The authentication procedure between Ambient IoT device and application is outside the scope of 3GPP.

*** END OF CHANGES ***