3GPP TSG-SA3 Meeting #119AdHoc-e	S3-250152

# Online, Electronic meeting, 13 -16 January 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	Conclusion for KI#3 Security aspects of digital asset container in 5G

Document for:	Approval

Agenda item:	5.18

Spec:	3GPP TR 33.721

Version:	0.5.0

Work Item:	FS_Metaverse_Sec

Comments

<Proposals, reason for change, abstract, comments if necessary (optional)>

This pCR proposes conclusion for KI#3 on Security aspects of digital asset container in 5G.

The TS 23.438 [8] specifies in sufficient detail the requirements, architecture, procedures and APIs for digital asset management, the digital asset service is part of the SEAL services specified in 3GPP TS 23.434 [7].

The solutions in the present TR propose extending the existing security mechanism for SEAL or CAPIF to authenticate and authorize a digital client for digital asset creation and access. Moreover, digital asset can be signed by DA server and RNAA can be reused to get authorization from the owner of digital asset.

Proposed Changes

* * * First Change * * * *

<Proposed change in revision marks>

# 7	Conclusions

## 7.3	Conclusions for Key issue #3: Security aspects of digital asset container in 5G

The TS 23.438 [8] specifies in sufficient detail the requirements, architecture, procedures and APIs for digital asset management, the digital asset service is part of the SEAL services specified in 3GPP TS 23.434 [7].

The normative work of the KI#3 aligns with the architecture of digital asset management defined in the TS 23.438 [8] and the below security solution principles should be followed:

- Security for the SEAL interfaces, especially SEAL-S, SEAL-UU, specified in TS 33.434 [4] is applied for protection of digital asset management interfaces such as DA-S, DA-UU.

- When CAPIF is used as specified in TS 23.434 [7], the security mechanism for CAPIF specified in TS 33.122 [5] shall be followed.

- VAL UE authorization mechanism specified in TS 33.434 [4] is applied for authorization of DA-C for digital asset access.

- When CAPIF is used as specified in TS 23.434 [7], API Invoker authorization mechanism specified in TS 33.122 [5] applied for authorization of DA-C for digital asset access. RNAA is reused for getting authorization from the resource owner in case the resource owner is associated with a UE which can be identified by CCF.

Editor’s Note: authentication and authorization of (VAL) user for digital asset access is FFS.

Editor’s Note: Additional conclusions is FFS.

* * * End of Changes * * * *