3GPP TSG-SA3 Meeting #119AdHoc-e	S3-250011

# Online, Electronic meeting, 13 -16 January 2025

Source:	Sony

Title:	KI#3, Conclusions

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.5.0

Work Item:	FS_AIOT_Sec

Comments

The Ambient IoT Check point at TSG SA#106 resulted in Way Forward in SP-241980.

The Way Forward includes important aspects on how to focus the work in Q1 to reach conclusions that can be implemented during the normative phase during Q2 and Q3, 2025.

The most important guidance is to focus the work on device type 1 (the most constrained device type) and topology 1 (base station reader), as this is what RAN will specify in Rel-19 according to approved WID in RP-243326.

In TR 33.713 v.0.5.0 there are 18 solutions documented related to KI#3 Privacy by protecting AIoT device identifiers. Solutions addressing KI#3 shall provide “Mechanisms for mitigating privacy threats (described above) by identifying, linking, and tracking the identifiers of AIoT Device(s) shall be supported”.

Proposal 1: As device type 1 is very constrained it can be concluded that these devices cannot use an USIM. Solution 11 and 12 cannot be considered.

SA#106 is specifically asking SA3 whether to use a temporary ID or not. Temporary ID is an ID used to hide the permanent ID of the AIoT device when the network addresses the AIoT device. The solutions documented in TR 33.713 use different names for this ID: Temporary ID, Protected ID or Concealed ID, but the all serve the same purpose - to hide the Permanent ID. The differences between the solutions are how and when the network/device derives the ID.

Solutions 20, 21, 22, 30, 35 and 38 require the AIoT device to perform computational tasks after receiving the paging/trigger message to detect that the paging/trigger message was intended for the device. Such paging/triggering message will wake up all AIoT devices within the Reader’s range and all AIoT devices will start executing computational tasks. This means that AIoT devices will waste their power even if the paging/trigger message was not intended for them, resulting in that it might not be possible to reach them for some time until enough power has been harvested again.

Observation: To not deplete the AIoT device’s limited power storage, the computational tasks must consume less power than what the AIoT device can harvest from the paging/trigger message or RF signal accompanying the paging/trigger message. The AIoT device must have enough power remaining after these tasks to respond in case the AIoT device was paged/triggered. There is no evidence that the power available to harvest from the paging/trigger message meets the power budget of the computational tasks.

Proposal 2: Not consider solutions 20, 21, 22, 30, 35 and 38 as there is high risk that such solution will result in situations the device will not be able to respond when actually paged/triggered.

Solutions 23, 31 and 34 includes either the full permanent ID or partial permanent ID (Filter info or group ID) in the paging/trigger message. The response includes a protected ID. In case the response can be linked to the paging/message, then the permanent ID/partial permanent ID is exposed.

Observation: As device type 1 is using backscattering to respond, it will be possible to link the response to the trigger message. As the trigger message included the permanent ID or partial permanent ID these solutions cannot provide privacy protection.

Proposal 3: Not consider solutions 23, 31 and 34.

The remaining solutions: 13, 24, 25, 26, 27, 28 and 29 require non-volatile memory (NVM) and can be divided into two groups:

- Network assigned temporary ID in an encrypted message: solution 24 and 29.

- Network provided security parameters allowing AIoT ID to derive temporary IDs locally: Solutions 13, 25, 26, 27 and 28.

Group 1: Require more messages exchanged between the AIoT device and network After the first pair of messages have been exchanged a second pair of protected messages including the encrypted new temporary ID is exchanged.

Group 2: The second pair of messages as required for group 1 is not needed. But there is a risk that the algorithm used the device may get out-of-synch with the network algorithm.

- Solution 13 proposes to always provide a new Nonce to the device.

- Solution 25 proposes that when the network detect the out-of-synch case and in that case restarts using the default ID.

- Solution 26 proposes that when the network to detect the out-of-synch case and a new seed is provided to the device.

- Solution 27 and 28 do not provide any solution to the out-of-synch case.

Observation: Use of NVM increases the complexity of the device and writing to the NVM requires energy. This means that a device shall not engage in AIoT communication if it has not harvested enough power to perform both communication and writing to the NVM.

Proposal 4: As group 1 above requires an extra pair of encrypted messages exchanged compared to group 2, it is proposed to not select solutions out of group 1.

Proposal 5: It is proposed to select baseline from the remaining solution to 13, 25, 26, 27 and 28 in Group 2.

Observation: Solution 13 request message includes many parameters: Temporary ID, Nonce, SQN, Command and MAC. The Assumption in RAN2 is that the paging message includes ID(s). There is also an EN in the solution description – “Whether Nonce in Step 6 can be sent to the device depends on RAN paging message”.

Observation: Solution 25 adds one additional message at the end of the procedure in which the network provides the device with a new freshness parameter.

Observation: Solution 26 does not have the extra parameters in the paging message, nor does it include an extra message. However, the details of the Temporary ID derivation algorithm are deferred to the Normative phase.

Observation: Solution 27 does not have the extra parameters in the paging message, nor does it include an extra message. However, the solution does not address the out-of-synch issue.

Observation: Solution 28 uses always the same encrypted device ID in the paging message. The response is a HASHed device ID based on an indexed HASH table. There is a security risk that a rouge Reader can replay the encrypted device ID to trigger the AIoT device multiple times to respond the HASHed IDs in the table. The solution does not address the out-of-synch issue of Index value.

Proposal 6: Conclude that the normative work can be based on solution 26 and that the algorithm will be detailed during the normative phase.

Proposed Changes

* * * First Change * * * *

## 7.x	Conclusions on Key Issue #3

For key issue #3 (Privacy by protecting AIoT device identifiers) described in clause 5.3, it is concluded that the normative work is based on solution 26 and that the Temporary ID derivation algorithm will be detailed during the normative phase.

* * * End of Changes * * * *