3GPP TSG-SA3 Meeting #119AdHoc-e	S3-250087

# Online, Electronic meeting, 13 -16 January 2025

Source:	Samsung

Title:	Updates to conclusion for key issue#2

Document for:	Approval

Agenda item:	5.18

Spec:	3GPP TR 33.700-22

Version:	0.3.0

Work Item:	FS_CAPIF_Ph3-sec

Comments

This pCR proposes to update the conclusion for key issue#2 on CAPIF interconnection security.

Proposed Changes

* * * First Change * * * *

## 7.2	Conclusion for KI #2: CAPIF interconnection security

It is concluded that for CAPIF-6 and CAPIF-6e reference points, same security mechanisms specified in clauses 6.6 and 6.10 of TS 33.122 [4] for CAPIF-3/4/5 and CAPIF-3e/4e/5e reference points will be used, respectively.

For security method negotiation procedure (as per requirement 2), clause 6.3.1 in TS 33.122 [4] can be re-used with the following enhancement:

- The API invoker includes the CCF information in the Security method request message.

- API invoker is provided with the CCF information by the on-boarded CCF-A.

For mutual authentication between API invoker (on-boarded to CCF-A) and the AEF (registered to CCF-B), the procedures as defined in clause 6.5.2 of TS 33.122 [4] can be re-used with the following enhancement:

- When using TLS-PSK or PKI:

  - The API invoker includes the CCF information in the Authentication Initiation request message.

  - AEF uses the CCF information to retrieve the security materials from CCF-A via CCF-B.

- When using TLS with OAuth token:

  - CCF-B provides an access token to the API invoker via CCF-A as specified in clause 6.5.2.3 in TS 33.122 [4]. The access token is specific to the AEF, which is providing services in domain B and signed by the CCF-B as per IETF RFC 7515[12]. AEF validates the access token using CCF-B’s certificate.

Editor’s note: Further conclusions are FFS.

* * * * End of Changes * * * *