3GPP TSG-SA3 Meeting #119AdHoc-e	S3-250142

# Online, Electronic meeting, 13 -16 January 2025

Source:	OPPO

Title:	Update Solution 18 in TR 33.713

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.5.0

Work Item:	FS_Ambient_IoT_Sec

Comments

This pCR proposes to update solution 18 for FS_AIOT_SEC in TR 33.713.

Proposed Changes

* * * Start of Change * * * *

## 6.18	Solution #18: Authorization procedure for AF-based intermediate node selection

### 6.18.1	Introduction

This solution is proposed to address Key Issue #2, supporting the authorization for AF-based intermediate node selection. This solution applies to Topology 2.

The Inventory/Command procedure described in solutions (e.g. #11, #33, etc.) of TR 23.700-13 [2] is used to describe the UE authorization mechanism.

The authorization of intermediate node is based on the UE subscription data stored in the UDM. Once receiving the potential intermediate node information provided by the AF, the AIoTF interacts with its UDM to obtain the UE subscription data. The AIoTF determines whether the UE is authorized to provide the requested Ambient IoT service by using the Ambient IoT service information, Ambient IoT service area information, and/or Ambient IoT device information included in the UE subscription data.

### 6.18.2	Solution details

Figure 6.18.2-1: Authorization procedure for AF-based intermediate node selection

1.	The AF sends the Inventory/Command request to AIoTF via NEF. The Inventory/Command request may include UE ID(s), Ambient IoT device ID, area information, command, etc.

2.	For each UE as the potential intermediate node provided by the AF, the AIoTF interacts with its UDM to obtain the UE subscription data for UE authorization.

3.	For each UE, the AIoTF checks the UE authorization as follows:

-	If the Ambient IoT service information is included in UE subscription data, the AIoTF determines whether the UE is authorized to provide the requested Ambient IoT service.

-	If the Ambient IoT service area information is included in UE subscription data, the AIoTF determines whether the UE is authorized to provide the requested Ambient IoT service.

-	If the Ambient IoT device information is included in UE subscription data, the AIoTF determines whether the UE is authorized to provide the requested Ambient IoT service.

If multiple UEs are authorized, the AIoTF further decides to select one or more UE(s) as intermediate node(s).

4.	For the selected UE, the AIoTF sends the inventory/command request to the UE.

Note:	Alignment of the details of subscription information to the architecture design is not addressed in the present document.

### 6.18.3	Evaluation

Note: No further evaluation is expected in the present document.

This solution addresses the requirement of Key issue #2.

This solution is applicable for both Inventory and Command procedures.

This solution allows the AIoTF to check the UE authorization by using the UE subscription data. The AIoTF obtains the identifier of candidate UE Reader from AF and interacts with the UDM to obtain its subscription data.

In this solution, the granularity of UE authorization can be Ambient IoT service level, Ambient IoT device level, or area level.

* * * End of Change * * * *