3GPP TSG-SA3 Meeting #119AdHoc-e	S3-250155

# Online, Electronic meeting, 13 -16 January 2025

Source:	Ericsson

Title:	Update KI#1: Third party specific user identities to include NEF-AF interface security

Document for:	Approval

Agenda item:	5.2

Spec:	3GPP TR 33.790

Version:	1.0.0

Work Item:	FS_NG_RTC_SEC_Ph2

Comments

SA2 has worked on CRs on TS 23.228 for the aspect of the 3rd party identities in IMS.

One of the proposed changes for this feature is a NEF-AF interface for an AF to provision Rich Call Data (RCD), RCD addresses or RCD URLs for a specific IMPU or a wildcard IMPU. The security of this new procedure is not captured in the Key Issue #1: "Third party specific user identities" in TR 33.790.

This pCR updates the key issue with the NEF-AF security considerations.

Moreover the document proposes editorial changes and replacement of the normative language with informative language.

Proposed Changes

* * * First Change * * * *

## 5.1	Key issue #1: Third party specific user identities

### 5.1.1	Key issue details

According to TR 23.700-77 [2], there are scenarios that the third party subscribers use third party IDs (e.g., enterprise employee ID). The IMS network can present the third party ID to the callee during subsequent calling process. The third party subscriber can access the IMS network directly or via a SIP trunk as well.

From the security point of view, the enhanced IMS network needs to be able to support the identity verification and authorization of third-party user during an IMS call.

In addition, the originating IMS system potentially includes information for supporting third party identity verification and authorization. External Application Functions (AFs) can update this information via a NEF interface and therefore the security of the NEF-AF interface needs to be considered.

### 5.1.2	Threats

A malicious UE can use IDs belonging to others or forged IDs to initiate IMS calls in the IMS network.

A malicious UE can use an ID that no longer belongs to it to initiate IMS calls in the IMS network (e.g., the user use the ID allocated by a particular company even after leaving it).

The  transfer of the ID between IMS networks can  be manipulated by intermediary network entities. Consequently, the callee can receive a wrong ID.

If a malicious/unauthorized AF can modify the 3rd party identity related information stored on the originating IMS system then unauthorized users can impersonate users belonging to a legitimate organization.

### 5.1.3	Potential security requirements

The IMS system shall be able to coordinate with the third party to verify and authorize the third-party specific user identities.

The IMS network shall be able to support the integrity protection of the third-party specific user identities on the originating side and terminating side.

The IMS network shall support the authentication and authorization of Application Functions (AFs) creating/modifying/deleting the third party related information maintained on the originating IMS system.

* * * End of Changes * * * *