3GPP TSG-SA3 Meeting #119AdHoc-e	S3-250162

# Online, Electronic meeting, 13 -16 January 2025

Source:	Nokia

Title:	KI1.1 Solution 3 update

Document for:	Approval

Agenda item:	5.19

Spec:	3GPP TR 33.700-22

Version:	0.3.0

Work Item:	FS_CAPIF_Ph3-sec

Comments

Update to solution 3 for alignment: ROC -> ROF.

Proposed Changes

* * * First Change * * * *

### 6.3.2	Solution details

If the CAPIF provider domain administrator decides to use GBA, procedures for mutual authentication and secure session establishment between ROF and CCF follow the clause 4 in TS 23.220 [8].

If the CAPIF provider domain administrator decides to use AKMA, procedures for mutual authentication and secure session establishment between ROF and CCF follow the clause 6 in TS 33.535 [9].

If the CAPIF provider domain administrator decides to use TLS, certificate based mutual authentication is performed between the ROF and CCF using TLS. Certificate based authentication shall follow the profiles given in 3GPP TS 33.310 [6], subclauses 6.1.3a and 6.1.4a. The structure of the PKI used for the certificate is out of scope of the present document. TLS shall be used to provide integrity protection, replay protection and confidentiality protection. Security profiles for TLS implementation and usage shall follow the provisions given in TS 33.310 [6], Annex E.

Editor’s Note: The authentication method to be used in TLS is FFS.

Editor’s Note: Whether GBA, AKMA and TLS need to be simultaneously supported is FFS.

Editor’s Note: How the solution works for the ROF that can be a web browser is FFS.

* * * End of Changes * * * *