3GPP TSG-SA3 Meeting #119AdHoc-e	S3-250216

Online, Electronic meeting, 13 -16 January 2025						merger of S3-250023, S3-250169

# Source:	Huawei, HiSilicon, Nokia

# Title:	editorial corrections in clauses 6.1/6.2 and 6.10

# Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

Approve the pCR proposal for TR 33.700-22

# 2	References

[1]	3GPP TR 33.700-22

# 3	Rationale

Corrections to typos and implementation errors are proposed for clause 6.1/6.2 and 6.10.

# 4	Detailed proposal

pCR

***  START OF FIRST CHANGES  ***

## 6.1	Solution #1: Security protection mechanism for CAPIF-8 reference point

### 6.1.1	Introduction

This solution is for KI #1.1 and addresses the security requirements for protecting CAPIF-8 reference point. This solution proposes to use TLS to protect CAPIF-8 reference points.

### 6.1.2	Solution details

TLS is to be used to provide integrity protection, replay protection and confidentiality protection. Security profiles for TLS implementation and usage follows the provisions given in TS 33.310 [6], Annex E.

### 6.1.3	Evaluation

This solution proposes to use TLS to provide security protection mechanism for CAPIF-8 reference point. No new security protection mechanism needed to be defined.

## 6.2	Solution #2: CAPIF-8 reference point security

### 6.2.1	Introduction

This solution addresses "Key Issue #1.1: CAPIF-8 reference point ".

The resource owner function (ROF) interacts with the authorization function (AZF) in the CAPIF core function (CCF) through the CAPIF-8 reference point. This solution proposes mutual authentication between the ROF and AZF. Besides, the messages exchanged between them are protected with integrity protection, replay protection and confidentiality protection.

NOTE: The AZF is part of the CCF and is used interchangeably with the CCF.

### 6.2.2	Solution details

#### 6.2.2.1	Mutual authentication

For authentication between a ROF and an AZF/CCF, mutual authentication based on TLS is proposed. The CCF is authenticated by the CCF certificates. The certificate profiles follow the TS 33.310 [6], clause 6.1.3a. The ROF authentication can be based on the ROF certificate, the pre-shared key or password etc., and is left for implementation.

NOTE: The structure of the PKI used for the certificate is out of scope of the present document.

#### 6.2.2.2	Protection of messages between ROF – AZF/CCF

TLS is used to provide integrity protection, replay protection and confidentiality protection for the CAPIF-8 interface.

The security profiles for TLS implementation and usage follow the provisions given in the clause 6.2 of the TS 33.210 [7].

### 6.2.3	Evaluation

The solution addresses the requirements of Key Issue #1.1.

TLS based mutual authentication is performed between the ROF and the CCF to establish a secure channel. The security protections for all messages transmitted through the channel include integrity protection, confidentiality protection and replay protection.

The solution assumes the ROF can handle the credentials. It does not cover recovery in case that the ROF loses its credentials.

***  END OF FIRST CHANGES  ***

***  START OF SECOND CHANGES  ***

## 6.10	Solution #10: resource-level and/or API-level authorization and revocation

### 6.10.1	Introduction

This solution addresses "Key Issue #1.3: Finer granular authorization".

In this solution, authorization and/or revocation with finer granularity is proposed to support finer granularity of access control for a service API, e.g., service operation level access and/or resource level access.

NOTE: the supported granularity at the service operation level or the resource level will be specified in TS 23.222 [3].

### 6.10.2	Solution details

#### 6.10.2.1	Service operation/resource level authorization

The procedure for authorizing an API invoker by the CCF/AZF is described as follows:

1.  The API invoker sends an authorization request to the CCF/AZF. In addition to the API invoker ID, the authorization request includes the indication of the requested service operation/resource. For RNAA, the request also includes the GPSI of the UE.

2.	The CCF/AZF verifies the API invoker ID and the requested service operation/resource, if available, match information, e.g., subscription information, stored at CCF/AZF.

3.	The authorization result is sent to the API invoker if verification is successful. If a token is issued, the token claims include the granted service operation/resource, and GPSI for RNAA.

#### 6.10.2.2	Service operation/resource level revocation

The procedure for revoking authorization to an API invoker by the CCF/AZF is described as follows:

1.  The API invoker/ROF/UE sends a revocation request to the CCF/AZF. In addition to the API invoker ID, the revocation request includes the indication of the requested service operation/resource. For RNAA, the request also includes the GPSI of the UE.

2.	The CCF/AZF verifies the API invoker ID and, if successful, revokes the service operation/resource. The CCF/AZF updates its stored authorization information, e.g., updated service operation levels or resource levels.

3.	The CCF/AZF responses to the API invoker/ROF/UE. The response includes indication whether revocation is successful, and the updated authorization information, e.g., updated service operation levels or resource levels. If a token is issued, the token claims include the updated service operation/resource levels, and GPSI for RNAA.

### 6.10.3	Evaluation

This solution addresses the requirements of the Key Issue #1.3.

This solution provides an authorization procedure and a revocation procedure with finer granularity. Both procedures are reusing the mechanisms specified in TS 23.222 [3]. The additions are IEs corresponding to the granularity levels included in the authorization/revocation request/response messages. The granularity levels of authorization and/or revocation will be aligned to TS 23.222 [3].

***  END OF SECOND CHANGES  ***