3GPP TSG-SA3 Meeting #119AdHoc-e	S3-250090

# Online, Electronic meeting, 13 -16 January 2025

Source:	Samsung

Title:	Evaluation updates for solution#6

Document for:	Approval

Agenda item:	5.2

Spec:	3GPP TR 33.790

Version:	0.6.0

Work Item:	FS_NG_RTC_SEC_Ph2

Comments

This pCR proposes to update the evaluation for solution#6.

Proposed Changes

* * * First Change * * * *

### 6.6.3	Evaluation

This solution addresses the requirements of KI#2: Security of IMS based Avatar communication.

In this solution, BAR returns the avatar representation only when the avatar representation owner (i.e. UE1) requests its avatar. For that purpose, UE1 generates the attestation which BAR uses to verify whether the owner is requesting its avatar representation. In addition, the avatar representation which is privacy related information is end-to-end protected in receiving UE centric IMS avatar call flow. In other words, the UE1's avatar representation is not exposed to the IMS network of UE2, for protecting UE1's privacy.

This solution requires UE to generate an attestation with an avatar ID.

This solution requires DCSF to check whether the received avatar ID is in the UE-A’s avatar ID list.

This solution requires MF/DC AS to request the UE-A's avatar representation with UE-A’s attestation.

This solution requires BAR to determine whether the request for an avatar representation originates from the host of the avatar representation (i.e., UE-A) by verifying the UE-A's attestation.

* * * * End of Changes * * * *