3GPP TSG-SA3 Meeting #119	S3-245319

Orlando, US  11 – 15 November 2024	(revision of xx-yyxxxx)

# Source:	Motorola Mobility [Rapporteur]

# Title:	New WID on enablers for Zero Trust Security in SBA

# Document for:	Approval

# Agenda Item:	6

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: enablers for Zero Trust Security in SBA

Acronym: eZTS_SBA

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-19

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_ZTS | SA3 | 960038 | Study on Zero Trust Security principles in mobile networks |
| FS_eZTS | SA3 | 1020034 | Study on enablers for Zero Trust Security |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification: NA

# 3	Justification

Release-18 TR 33.894 [1] performed analysis of 5G core SBA security aspects and identified that authentication, authorization, and communication security were supported between NF service consumers and NF service producers by means of standard mechanisms specified in TS 33.501 [3]. But there is no standardization support available for data collection to enable operator to perform security evaluation and monitoring to identify potential NF compromise in advance to improve access control security at the SBA layer to prevent further threat lateral movement. Further TR 33.894 [1] clause 5.1.5.3 for the data collection and exposure aspect states that, ‘with not yet specified data collection, this information needs to be well defined and explicitly specified to allow for interoperability and secure operation of installed base’. Further Release-19 TR 33.794 [2] identified and concluded the following potential security scenarios where data collection and exposure can help to identify the threats/attacks on 5G SBA layer for KI#1 ‘Data exposure for security evaluation and monitoring’. Specific conclusion details can be referred in TR 33.794 Clause 8.1 [2].

The security incidents or scenarios in SBA where data can be collected in the SBA layer includes:

1)	authentication and authorization failure event;

2)	unexpected setup of TLS session and API invocation related to unauthorized reconnaissance;

3)	malformed message event;

4)	high service load;

5)	unexpected SBI call flows; and

6)  unexpected use of APIs exposed by services in SBA layer

It is concluded for KI#1, ‘Data exposure for security evaluation and monitoring’, that it will be addressed by requirements for data collection to enable security evaluation and monitoring on the SBA layer. No new interface nor protocol will be specified.

Further TR 33.794 [2] identified the potential security policy enforcement use-cases (i.e., NF service discovery, Access Token request, Service Request process, NF service update) for SBA access control decision enhancement related to KI#2 on ‘Security mechanisms for policy enforcement at the 5G SBA’ while also considering different communication models described in TS 23.501 Annex E [4]. Further the study states security requirement in Clause 6.2.3 [2] as ‘The 5GS should provide the means to configure suitable PEP within the 5G SBA with information about an NF that has been subject to an attack.’. But it is concluded that normative work is not needed hence this WID proposal recommends providing informative guidance in TS 33.501 [3] to describe the potential realization on security policy enforcement points within SBA to address the KI#2 security requirement.

References:

[1] 3GPP TR 33.894, "Study on applicability of the zero trust security principles in mobile networks", Release 18.

[2] 3GPP TR 33.794, "Study on enablers for Zero Trust Security", Release 19.

[3] 3GPP TS 33.501, "Security architecture and procedures for 5G System", Release 19.

[4] 3GPP TS 23.501, "System architecture for the 5G System (5GS)", Release 19.

# 4	Objective

The objectives of this work item include the following aspects:

(i) Data exposure for security evaluation and monitoring:

- Specify the system security requirements in TS 33.501 for the following aspects to support security event(s) data collection at 5G core SBA layer to enable security evaluation and monitoring. General requirements for security event logs are listed below.

1. The NF supports the generation of security event logs.

2. Security event logs at-rest prior to being collected are confidentiality and integrity protected.

3. Security event logs in-transit are confidentiality and integrity protected.

Security event logs contain security relevant events data. Requirements related to security relevant events data for monitoring. These requirements are optional for implementation:

a.	The NF collects information on the SBA layer about malformed messages it receives that deviate from the 3GPP specified messages or are considered invalid according to the protocol specification and network state.

b.	The NF collects information about events involving receiving a massive number of incoming messages on the SBA layer.

c.	The NF collects information about failed authentication and authorization attempts from inbound connection on the SBA layer.

d.	The NF collects information about potential replay attacks on the SBA layer.

e.	The NF collects information about potential abnormal SBI call flows as defined for the communication models in Annex E of TS 23.501 [4],

NOTE: The method for collecting this data from the NF is upto implementation

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| NA |  |  |  |  |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 33.501 | Security architecture and procedures for 5G system | SA#108 (June 2025) | NA |

# 6	Work item Rapporteur(s)

# 7	Work item leadership

Primary: SA3

Secondary: NA

# 8	Aspects that involve other WGs

NA

# 9	Supporting Individual Members

Supporting IM name

Accenture

AT&T

British Telecom

Cablelabs

Charter Communications

CISA ECD

Deutsche Telekom

Interdigital

Johns Hopkins University APL

MITRE

Nokia

OTD US

Peraton Labs

T-Mobile US

Telecom Italia

US NSA

Vodafone