3GPP TSG-SA WG3 Meeting #119 	S3-244608

11 – 15  November 2024 Orlando, USA                                                     revision of S3-20xabc

# Source: 	Interdigital

# Title: 	Additional conclusions for KI#1

# Document for: 	Approval

Agenda Item: 	5.7

# 1	Decision/action requested

Approve the pCR to TR 33.700-29

# 2	References

[1]	TR 33.700-29 Study on Security Aspects of 5G Satellite Access in the 5G architecture

# 3	Rationale

This contribution proposes additional text explaining the approach for enhancements to provide protection against DoS attacks before a security context is established.

*** Start of 1st Change ***

## 7.1	Conclusions for Key Issue #1: Security protection in Store and Forward Satellite Operation

For the HSS on the satellite deployment, the following conclusions are made:

-	The normal security procedures are used as security can be established using legacy security procedures (e.g. AKA, NAS security mode command, etc.) using the security credentials in the HSS onboard the satellite;

-	As an option for the operator, IOPS-based keying which is captured in the informative Annex F in TS 33.401 [3] can be used to limit the impact of exposing the long-term key in the HSS in the satellite; and

NOTE:	Any related enhancements of IOPS-based keying agreed during the normative phase are to be captured in the informative Annex F in TS 33.401 [3].

-	The security of communications between the proxies on satellite and ground is out of 3GPP scope.

The following principles are agreed for supporting Store and Forward operation with a split MME architecture:

-	Mutual authentication between the UE and the 3GPP network in S&F operations may require the involvement of more than one MME on-board a satellite, in which case the ground network is responsible for the selection and provisioning of MMEs on-board the same, or another satellite with the necessary information to perform or finish a mutual authentication procedure.

-	It has to be ensured that the latest NAS security context of the UE is available at the MME on-board which processes the NAS security;

-	The security of communications between the proxies on satellite and ground is out of 3GPP scope.

-	Enhancements to provide protection against DoS attacks before a security context is established are to be determined during normative work based on the solution(s) captured in this document. These solutions describe two distinct approaches, one of them requiring the provisioning of satellite/UE-specific credentials and capable of stopping DoS attacks, and another one that does not require satellite/UE-specific credentials and is capable of slowing down DoS attacks while disproportionally affecting the DoS attacker. The comprehensive remediation will require combining the two approaches as demonstrated in Solutions ##33, 34, and 35.

Editor’s Note: Further conclusions are FFS.

*** End of 1st Change ***