3GPP TSG-SA3 Meeting #119	S3-244632

Orlando, US, 11 -15 November 2024

# Source:	3GPP CT4

# Title:	Reply LS on clarification on home network triggered re-authentication

# Document for:	Information, Discussion

Agenda Item: 3

3GPP TSG-CT WG4 Meeting #125	C4-244496

# Hefei, China; 14th – 18th October 2024

# Title:	Reply LS on clarification on home network triggered re-authentication

# Response to:	LS S3-243412 on clarification on home network triggered re-authentication

# Release:	Rel-18

# Work Item:	HN_Auth

Source:	CT4

To:	SA3

Cc:

Contact Person:

Name:	Shuang Liang

Tel. Number:	+86 15850539616

E-mail Address:	liang.shuang3@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

CT4 would like to thank SA3 for their reply on CT4’s questions. As indicated in reply LS:

For case #5 is a failure case and the UDM can take immediate action as already specified in 33.501, 6.1.5 (e.g. try AMF of another access).

However, CT4 notices the behaviour of the AMF defined in clause 6.1.5.2 of TS 33.501 may cause confusion.

3. 	After receiving the Nudm_UECM_Re-AuthenticationNotification  message from the UDM, the AMF/SEAF shall decide whether to run the primary authentication procedure based on its own local authentication policy, and the UE state (e.g. , if the UE is under handover, or if the UE is already under authentication by the AMF before receiving the authentication notification from the UDM). If the AMF/SEAF determines that it cannot run a primary authentication as described in step 4 (e.g., due to local policy), the AMF/SEAF sends the authentication response message to the UDM with a failure cause else it acknowledges the request. If the AMF/SEAF acknowledged the request but the AMF/SEAF is not able to initiate the primary authentication towards the UE (e.g. if UE is not reachable), the AMF/SEAF shall set the authentication pending flag. Upon receiving a failure from the AMF, the UDM may check if another AMF is available over the other access. If available, the UDM may select another AMF and retry Step 2.

During the discussion, CT4 considers the following scenarios:

When the AMF receives a Reauth Notifications request for a UE with a valid context, the AMF cannot page the UE immediately, e.g. caused by long DRX, the AMF sends an acknowledge (i.e. not a failure response) immediately. As defined in the current SA3 specification, the UDM will not re-try re-authentication towards to another AMF if available.

In order to ensure the re-authentication procedure is performed as soon as possible, CT4 is proposing that the AMF indicates reauthentication-pending in the acknowledge. Then the UDM decides whether to retry re-authentication in another AMF (if available) based on the indication.

CT4 would like to inform SA3 that in CT4#126 CT4 will work on the solution proposed. Unless SA3 considers that CT4’s proposal conflicts any principle specified in SA3, CT4 will address this proposal in stage3 specification and, when completed, CT4 will inform SA3 to align 3GPP TS 33.501 accordingly.

2. Actions:

To SA3

ACTION:

CT4 kindly asks SA3 to consider CT4 proposal and provide a response if the proposed solution is not acceptable by SA3.

3. Date of Next CT4 Meetings:

CT4#126		November 18-21, 2024 				Orlando, US