3GPP TSG-SA3 Meeting #119	S3-244864

Orlando, US  11 – 15 November 2024													   revision of

Title:	LS reply to LS on RAN2 agreements and assumptions for Ambient IoT

Response to:

Release:	Release 19

Work Item:	FS_Ambient_IoT_Sec

Source:	Nokia (to be SA3)

To:	RAN2

Cc:	SA2

Contact Person:

#### Name: Bo Bjerrum

#### E-mail Address:	bo dot bjerrum at nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA3 would like to thank RAN2 for their LS concerning RAN2 agreements and assumptions for Ambient IoT and provide the following feedback.

SA3 has analysed the access procedures and identified, solutions with less than 3 steps e.g. the 2-step access resolution approach has insufficient steps to establish a security context, fulfilling all security requirements/primitives for KI#4 and KI#5 in TR 33.713, prior to sending the upper layer data. SA3 doesn’t mandate to support all the primitives, which implies less than 3 steps can be applicable in some cases.

Furthermore, it has been identified, that the AIoT device assigns a random identifier as part of both 2 and 3-step contention resolution, which will be used as part of the contention resolution send back from the reader. As the identifier is not protected, SA3 recommends that the identifier is assigned randomly without any relation to the real identifier nor upper layer data,

In relation to the feasibility of the command only use case, SA3 would like to provide the following answer; As the security establishment is part of the inventory/contention resolution, which enables secure communication of commands, it’ll make this communication unsecure if the inventory/contention resolution is omitted. For this reason, SA3 doesn’t see the “command-only“ use case feasible with the given access procedures.

2. Actions:

To SA2:

SA3 kindly asks RAN2 to take above information into account.

3. Date of Next SA3 Meetings:

TSG-SA WG3 Meeting #120	17-21 February 2025	Athens, GR

TSG-SA WG3 Meeting #121	7-11 April 2025	Goteborg, SE