3GPP TSG-SA3 Meeting #119	S3-244919

Orlando, US  11 – 15 November 2024													   revision of

# Source:	Nokia

# Title:	Public key or cert retrieval for access token verification

# Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

Discussion on CR proposal for adding a new operation to a rel-15 service.

# 2	References

[1]	3GPP TS 33.501

# 3	Rationale

CR S3-XXXXX proposes to enhance SBA service “Nnrf AccessToken” by a new operation to retrieve the certificate/public key for access token validation.

We are at the end of releases for 5G. Hence, from Rel-15 to Rel-18 the problem of certificate/key distribution/retrieval must have been handled so far in an implementation / operator specific way. Hence it was questioned, what is the real benefit to introduce in the late 5G an additional service or service operation.

In the following, the pros and cons are analysed.

1. In which situation a public key of issuer NRFs should be already available at NFp?

- NFp must be configured with a root CA certificate that is the base of any NRF leaf cert that is allowed to issue access tokens.

- An NFp registered to one NRF should also know the public key/cert of this NRF if the NRF is also issuing tokens.

- It can be also assumed that the recipient of a token (NFp) is configured with NRFs pub key / certificate information, if these NRFs are allowed to issue tokens for NFp’s services.

- It can further be assumed that a NRF has not only one pub key / certificate, but several for redundancy or in case of revocation.

Observiation 1: NFp is pre-configured with all information necessary to validate access tokens. Even if the token is not issued by the NRF where the NFp is registered, the NFp should be able to validate the access token with the pre-configured information assumed to be available at NFp.

2. In which situation could the token recipient have not sufficient information to allow validation of the token with the existing stored public key?

- The issuer NRF could have installed a new certificate, which was then used to sign the access token but not distributed to NFps.

Observation 2: In an operational good managed system, such situation should not appear. If the NRF has a change in certificate, all NFs previously pre-configured should be updated with the new certificate.

3. Possible use cases that require NFp action?

- The NFp was not configured with the certificate from an issuer NRF.

- The NFp receives an access token but has an old certificate from the issuer NRF.

- The operator has failed to provide an update to the NFp.

- The NFp failed to validate a cert.

Observation3: In above cases the so far proposed new service seems to be not sufficient to address the problem. Addressing several public keys of the same NRF. Error handling / reporting missing. Subscription for notifications as a baseline for retrieval of pub key/cert beforehand.

4. Where would NFp get the information from? NRF or other entity?

NFp could get this information direct from the issuer NRF, from a dedicated NRF for this purpose, or from any other entity like a certificate manager, etc.

Reasoning: SBA is the baseline since Rel-15. So far key distribution management has been handled implementation specific and depending on the operator policy.

Obervation 4: Operators have used processes for certificate management fromRel-15-Rel18 to manage this issue for all network functions. This has been so far kept implementation specific and out of scope in 3GPP.

5. In case a standardized method is needed, it is to be debated whether this will be a new service of an NRF or a new NF or any other entity? Or whether this will be an addition of a new operation to existing service at NRF?

If one wants to introduce such service or operation with NRF, there are several services possible to enhance, which would require studying.  Existing services are Nnrf_AccessToken (OAuth2 Authorization), Nnrf_NFManagement, Nnrf_NFDiscovery, and Nnrf_Bootstrapping.

a) AccessToken: is for operations directed to NF Service Consumers. Would this be the right service to address new actions needed by Service Producers?

b) NFManagemet: Key/certificate distribution or retrieval - it this rather an operational issue? The Nnrf_NFManagement service allows an NF, SCP or SEPP Instance in the serving PLMN to register, update or deregister its profile in the NRF.

c) NFDiscovery: discovers the set of NF Instances (and their associated NF Service Instances), represented by their NF Profile, that are currently registered in NRF and satisfy a number of input query parameters.

Could this be used for also discovering for information about a issuer NRF?

d) Bootstrapping: to let NF Service Consumers of the NRF know about the services endpoints it supports. Can be used in inter-PLMN scenarios.

e) New NRF service operation – dedicated for key/cert retrieval.

f) New network function - used for managing in general any of cert management issues, since OAuth token validation is only one of the situations where cert/pub key management is needed.

g) OAM mechanism.

Observation 5: If SA3 decides to standardize a mechanism for something that must be well established for 4 releases, careful analysis is needed to decide, if it would a NRF service, a NRF service operation to an existing service, a new NF or any other entity taking care of the cert/pub key distribution management. SA3 could also decide to document all different options in an informative annex.

6. Should 3GPP version of the OAuth token have a mandatory IE with URI? The argument of topology hiding was given during the discussion as a “no go” for this proposal, but why would there be topology hiding within an operator network?

Observation 6: If the NFp does not know the pub key/cert, mandating where to fetch the pub/key to be part of the Oauth token should be considered, such that (as of now) any proprietary mechanism can be used to retrieve the missing information.

7. Oauth 2.0 allows to include a URL, such that the issuer can indicate where to find the issuer’s certificate. Currently this is not mandated but could be beneficial for implementing vendor specific solutions without the need of a new service or service operation. The following fields are part of OAuth 2.0 RFC, but are not mandated

"x5u" (X.509 URL) Header Parameter - topology hiding is not an issue within one operator domain and the access token will be issued always from an NRF related to the producer domain

"x5c" (X.509 Certificate Chain) Header Parameter – potentially a size problem, but not necessarily.

"x5t" (X.509 Certificate SHA-1 Thumbprint)  Header Parameter - to identify a certificate but this cannot provide the certificate.

"x5t#S256" (X.509 Certificate SHA-256 Thumbprint) Header Parameter - to identify a certificate but this cannot provide the certificate.

Observation 7: mandating x5u in OAuth token may be a way forward.

# 4	Detailed proposal

A proper analysis of the situation on all potential implications of introducing normative text on public key / cert distribution needed before jumping to any conclusion.

For discussion, if an informative annex describing different ways/solutions should be created.