3GPP TSG-SA3 Meeting #119	S3-244949

Orlando, USA 11 – 15 November 2024

# Source:	Qualcomm Incorporated

# Title:	Discussion on incoming SUCI LS

# Document for:	Discussion

Agenda Item:	3.1

# 1	Decision/action requested

This contribution discusses the incoming LS on SUCI.

# 2	References

[1] 	C1-245039/S3-244629

# 3	Rationale

In their LS [1], CT1 have sent an LS to SA3 about error cases when calculating SUCI on the USIM. The error cases arise when the USIM has not been provisioned with the correct information to calculate a SUCI. A SUCI calculation on the USIM requires the USIM to be provisioned with the following parameters:

Home network identifier,

Routing indicator,

Home network public key,

Home network public key identifier, and

Protection scheme identifier

SA3 already deal with one of such error cases, i.e. the SUCI can be calculated with the NULL scheme if the USIM is not provisioned with a home network public key. Having not been provisioned with the other information is essentially the same error by the operator and it would seem much simpler to treat all these cases the same. Continuing to use 5G also means that the operator could detect that the error as the subscription is using NULL scheme when it should not.

Looking at other possibilities of behaviour. Firstly the UE could stop using 5G only. In such a case the UE would be forced to use 4G or earlier. This means that not only would the user’s IMSI be sent in the clear but the UE would lose the benefit of all the other additional privacy gains in 5G, e.g., initial NAS security and mandating GUTI changes after paging. So in this case the UE would end up with worse privacy than using the NULL scheme in 5G. Also the wrong configuration would not be directly noticeable to the home operator.

A second option would be to stop all 3GPP service with this UE. This would be a drastic option and stop the phone working. This means that any recovery would need to happen via the operators call centre. It would also prevent over the air updates of the USIM (this is also true for 5G only devices in the above case). This also seems wrong in that a 5G configuration error prevents the use of other accesses.

For devices with hard to change subscriptions, e.g. an inaccessible USIM, full recovery by either switching 5G or all coverage off at the UE would be much more difficult.

In summary, we believe that clear UE behaviour is needed for this case, so all phone acts the same, and the most straight forward in terms of balancing availability and privacy is to have the UE to use the NULL scheme after such an error from the USIM.

# 4	Detailed proposal

It is proposed to use S3-244950 as a response for the incoming LS.