3GPP TSG-SA3 Meeting #119	S3-245068

Orlando, US, 11 -15 November 2024

# Source:	Philips International B.V.

# Title:	Solution#31 update – addressing EN

# Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

SA3 is kindly requested to approve the changes proposed.

# 2	References

# 3	Rationale

With regards to Editor’s Note: Whether the lightweight function is a keyed hash function and how the device-specific key identifier is used are FFS.

The lightweight function may be a keyed hash function, as described in step 3. Furthermore, the AIoT MF knows which AIoT device(s) it is paging (e.g., whether it is a specific device or a group of devices), as such, the AIoT MF knows which credentials it needs to use to compute the expected AIoT device pseudonyms, hence, a device-specific key identifier is not required in step 4.

# 4	Detailed proposal

# *** START OF CHANGES ***

## 6.31	Solution #31: Ambient IoT ID privacy

### 6.31.1 Introduction

This solution addresses KI#3 and proposes a solution to protect the privacy of the identifier of an Ambient IoT (AIoT) device. The solution focuses on the functionality in the AIoT device:

- verifying a request to share the identity of the AIoT device,

- protecting the identity of the AIoT device, when sharing it.

The procedure fits the baseline RAN procedure agreed in RAN2#125bis:

Step A: Based on the service request, the reader sends the Initial Trigger Message indicating device(s) that need to respond;

Step B: Triggered device(s) performs the random access-like procedure, if needed;

Step C: The device may perform the data communication with the reader as needed.

### 6.31.2 Solution details

Figure 6.31.2-1:  Ambient IoT ID privacy

In step 0, AIoT device(s) are configured with identity(ies) and device specific parameters, such as security credentials including device- and group-specific credentials.

In step 1, the AIoT Management Function (AIoT MF) sends a message to the reader to trigger an inventory procedure including an inventory request message and a configuration associated with the inventory procedure. The configuration includes timers (e.g., to broadcast the request and to collect responses from AIoT device(s)), response aggregation requirements and forwarding criteria, etc.

Note 1: The AIoT Management Function (AIoT MF) depends on the entity that owns or manages the resource (i.e., AIoT device). If a device is owned or managed by the network, the disabling is triggered by a CN function, otherwise, it is triggered by a 3rd party Management Function managing the device.

In step 2, the reader UE/gNB sends the inventory request message, as received in step 1, to AIoT device(s). The inventory request includes a device (or group) identifier and a device-specific (or group-specific) challenge. Based on the configuration received, the reader UE/gNB may start a timer T1 to collect responses.

In step 3, The AIoT device processes the received request and checks whether the message:

3.1 is addressed to the AIoT device by matching its identifier (i.e., device or group identifier) against the received identifier, and

3.2 is sent by a trusted party by verifying the challenge value using its security credentials and identifiers.

If the checks 3.1 and 3.2 succeed, the AIoT computes a pseudonym based on its identifier and the received challenge value. This can be implemented by means of a lightweight function, e.g., a keyed hash function.

In step 4, the AIoT device sends its pseudonym in a response message (e.g., an inventory response message).

In step5, based on the configuration received in step 1, the reader UE/gNB aggregates the response message(s) received, until the time T1 runs out and/or other forwarding criteria (e.g., all device responses are received) are met.

In step 6, the reader UE/gNB forwards the response(s) to the AIoT MF.

# *** END OF CHANGES ***