3GPP TSG-SA3 Meeting #119	S3-245292

# Orlando, USA  11 - 15 November 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	Solution for KI#4 to authorize avatar by metaverse service provider

Document for:	Approval

Agenda Item:	5.18

# 1	Decision/action requested

Approve the solution added to TR 33.721

# 2	References

[1]	3GPP TR 23.700-21 Study on Application enablement architecture for mobile metaverse services

[2]	3GPP TR 33.721 Study on security aspects of 5G Mobile Metaverse services

[3]	3GPP TS 33.501 Security architecture and procedures for 5G system

[4]	3GPP TS 23.438 Service Enabler Architecture Layer for Verticals (SEAL); Digital Assets

[5]	3GPP TS 33.122 Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs

# 3	Rationale

Avatar and digital asset support, including digital asset avatar management and discovery, is discussed in Key issue #3 in TR 23.700-21, which requires to create, update, retrieve, delete and discover digital assets securely.

Correspondingly, KI#4 on Authentication and authorization of digital representation was approved and added in TR 33.721.

Architecture, procedures and information flows for digital asset service to support mobile metaverse services are specified in TS 23.438. Users can be associated with one or more digital assets like Avatars, software licenses, files, etc. Applications like mobile metaverse services can utilize the digital assets related to users, and the users can benefit from having the use of their digital assets between the various metaverse applications/platforms in an interoperable way. List of Metaverse service provider identifiers is part of digital asset profile.

In the current release of this specification, the digital asset type is limited to digital avatar.

This solution aims to support metaverse service provider to authenticate and authorize avatar when the users have the use of their avatars between the various metaverse service providers.

# 4	Detailed proposals

# * * * * First change* * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[2]	3GPP TR 23.700-21: "Study on Application enablement architecture for mobile metaverse services".

[3]	3GPP TS 22.156: "Mobile Metaverse Services; Stage 1".

[4]	3GPP TS 33.434: "Security aspects of Service Enabler Architecture Layer (SEAL) for verticals".

[5]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs"

[6]	3GPP TS 33.501: "Security architecture and procedures for 5G System"

[7]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows"

[x]	3GPP TS 23.438: “Service Enabler Architecture Layer for Verticals (SEAL); Digital Assets”

# * * * * second change* * * *

## 6.x	Solution #x: authorize avatar by metaverse service provider

### 6.x.1	Introduction

This solution addresses Key Issue #4 on Authentication and authorization of digital representation.

Avatar and digital asset support, including digital asset avatar management and discovery, is discussed in Key issue #3 in TR 23.700-21[2]. The corresponding requirements were defined in clause 5.4 of the TR 23.700-21[2] which include, for example, subject to operator policy, regulatory requirements and user consent, the metaverse enablement service shall provide digital asset management mechanisms as follows:

-	to create, update, retrieve, delete and discover digital assets securely.

-	to manage associations between digital assets and user identifiers.

-	to allow an authorized third party to manage digital asset(s) associated with a user.

Correspondingly, KI#4 on Authentication and authorization of digital representation was described in TR 33.721 which requires 5G system support authenticating a digital representation to represent a user in mobile metaverse services and authorizing the user/subscriber to use the digital representation (avatar) in mobile metaverse service.

Architecture, procedures and information flows for digital asset service to support mobile metaverse services are specified in TS 23.438 [x]. Users can be associated with one or more digital assets like Avatars, software licenses, files, etc. Applications like mobile metaverse services can utilize the digital assets related to users, and the users can benefit from having the use of their digital assets between the various metaverse applications/platforms in an interoperable way.

In the current release of this specification, the digital asset type is limited to digital avatar.

This solution aims to support metaverse service provider to authorize the user/subscriber to use the avatar when the users have the use of their avatars between the various metaverse service providers.

### 6.x.2	Solution details

Figure 6.x.2-1 Procedure to authorize usage of avatar in metaverse service

Precondition:

- Avatar is created and stored in digital asset server which included authorization information provisioned by the owner of the digital asset as part of DA profile, which includes owner list, allowed user list, allowed application list, spatial conditions, expiry time, etc., refer to clause 7.2 of TS 23.438 [x] for the detail authorization information in DA profile.

- Digital asset client retrieves avatar from the DA server.

Procedure:

1. DA client, on behalf of subscriber, sends metaverse service request to a metaverse service provider including avatar and avatar id.

Editor's Note: According to TS 23.438, DA client supports DA-UU interface towards the DA Server. Whether the DA client supports an interface towards the application server of the metaverse service provider is FFS.

2. The metaverse service provider sends request to DA server to check if the DA client on behalf of subscriber is allowed to use the avatar, the request includes avatar/DA id, UE id such as GPSI, application id of the DA client, application id of the metaverse service provider, location of the DA client or metaverse service provider, usage time.

3. The DA server retrieves DA profile based on DA id, and checks if the subscriber is in the allowed user id list, the application id(s) is in the allowed application list, the location is matched spatial conditions, and the usage time is before expiry time of the DA/avatar.

Editor's Note: The association of a subscriber with the user for digital asset is to be aligned with SA6.

4. The DA server returns the checking result to the metaverse service provider.

5. Metaverse service provider proceeds with the metaverse service request, e.g. show the avatar in spatial anchor/map, and sends response to the DA client.

### 6.x.3	Evaluation

TBA.

# * * * * End of changes * * * *