3GPP TSG-SA3 Meeting #119	S3-244661

Orlando, US, 11 -15 November 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.514 | CR | draftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add UDM SCAS test case for checking the authentication verification of a synchronization failure message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BSI (DE) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph3, SCAS_5G_Maint. |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A test case for detecting the processing of an unverified AUTS parameter from synchronization failure messages is not yet specified in the UDM SCAS tests, resulting in a lack of test coverage for key security features. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add UDM SCAS test cases to check if a synchronization failure message with an AUTS parameter containing an invalid MAC-S will be rejected. Subtest cases iterate of the 4 possibilities of input parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UDM SCAS tests may fail to detect a significant security flaw. This could result in attacks affecting the user authentication and privacy. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

4.2.2.X	Authentication verification of synchronization failure messages

Requirement Name: Authentication verification of synchronization failure messages

Requirement Reference: TS 33.501 [2], clause 6.1.3.3.2 and TS 33.102 [7], clause 6.3.5.

Requirement Description: The UDM shall check the range of the SQN and the MAC of the AUTS when it receives a synchronization fails. Depending on the range of the SQN and the validity of the MAC, the UDM shall respond accordingly.

Threat References: TR 33.926 [4], clause E.2.2.X, Synchronization verification failure.

Test Case:

Test Name: TC_VERIFY_AUTH_SYNC_FAIL_MSG_UDM

Purpose:

Verify that the UDM validates the authentication of synchronization failure messages.

Procedure and execution steps:

Pre-Condition:

- Test environment with one AUSF. The AUSF can be simulated.

- The tester has knowledge of the key material for a UE.

- The tester has knowledge of the algorithm used to increase the SQN and knowledge of how to determine the range 		of sequence numbers.

Execution Steps:

- If multiple SQN incrementing algorithms are supported by the UDM, the tester shall repeat this test case for all 			algorithms.

1. The tester sends a Nudm_UEAuthentication_Get and receives a response accordingly,

2. The tester computes the SQN from the AUTS by xoring the AK to the SQN_xor_AK part. The tester saves this 		SQN for later result comparison.

Test A: (SQN in range, MAC correct)

3. The tester generates a AUTS with the SQN in range of stored SQN number and a valid MAC-S.

4. The tester sends an Nudm_UEAuthentication_Get message to the UDM with a "synchronisation failure indication" and the generated parameters.

5. The UDM/ARPF performs steps 1-3 as described in TS 33.102 [7], clause 6.3.5.

Test B: (SQN out-of-range, MAC correct)

3. The tester generates an AUTS with an SQN that is out of range of the stored SQN number and a valid MAC-S.

4. The tester sends an Nudm_UEAuthentication_Get message to the UDM with a "synchronisation failure indication" and the generated parameters.

5. The UDM/ARPF performs steps 1-3 as described in TS 33.102 [7], clause 6.3.5.

Test C: (SQN in range, MAC incorrect)

3. The tester generates a AUTS with the SQN in range of stored SQN number and an invalid MAC-S, e.g., by randomly selecting the value.

4. The tester sends an Nudm_UEAuthentication_Get message to the UDM with a "synchronisation failure indication" and the generated parameters.

5. The UDM/ARPF performs steps 1-3 as described in TS 33.102 [7], clause 6.3.5.

Test D: (SQN out-of-range, MAC incorrect)

3. The tester generates an AUTS with an SQN that is out of range of the stored SQN number and an invalid MAC-S, e.g., by randomly selecting the value.

4. The tester sends an Nudm_UEAuthentication_Get message to the UDM with a "synchronisation failure indication" and the generated parameters.

5. The UDM/ARPF performs steps 1-3 as described in TS 33.102 [7], clause 6.3.5.

Expected Results:

For test case A, C, D the received SQN is in the same range as the original SQN.

In test case B, the received SQN falls within the range of the SQN sent to the UDM, but not within the range of the original SQN.

Expected format of evidence:

Suitable evidence for the interface could include a packet trace, such as a pcap file, accompanied by the Sequence Numbers (SQNs) and ranges recorded in a log file.

********** END OF CHANGE **********