3GPP TSG-SA3 Meeting #119	S3-244836

Orlando, US, 11 -15 November 2024

# Source:	Huawei, HiSilicon

# Title:	Adding evaluation to solution #10

# Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

Approve the pCR proposal for TR 33.700-22

# 2	References

[1]	3GPP TR 33.700-22

# 3	Rationale

This contribution provides evaluation to the solution #10.

# 4	Detailed proposal

pCR

***  START OF CHANGES  ***

## 6.10	Solution #10: resource-level and/or API-level authorization and revocation

### 6.10.1	Introduction

This solution addresses "Key Issue #1.3: Finer granular authorization".

In this solution, authorization and/or revocation with finer granularity is proposed to support finer granularity of access control for service API, e.g., service operation level access and/or resource level access.

NOTE: the supported granularity at the service operation level or the resource level will be specified in TS 23.222 [3].

### 6.10.2	Solution details

#### 6.10.2.1	Service operation/resource level authorization

The procedure for authorizing an API invoker by the CCF/AZF is described as follows:

1.  The API invoker sends an authorization request to the CCF/AZF. In addition to the API invoker ID, the authorization request includes the indication of the requested service operation/resource. For RNAA, the request also includes the GPSI of the UE.

2.	The CCF/AZF verifies the API invoker ID and the requested service operation/resource, if available, match information, e.g., subscription information, stored at CCF/AZF.

3.	The authorization result is sent to the API invoker if verification is successful. If a token is issued, the token claims include the granted service operation/resource, and GPSI for RNAA.

#### 6.10.2.2	Service operation/resource level revocation

The procedure for revoking authorization to an API invoker by the CCF/AZF is described as follows:

1.  The API invoker/ROF/UE sends a revocation request to the CCF/AZF. In addition to the API invoker ID, the revocation request includes the indication of the requested service operation/resource. For RNAA, the request also includes the GPSI of the UE.

2.	The CCF/AZF verifies the API invoker ID and, if successful, revokes the service operation/resource. The CCF/AZF updates its stored authorization information.

3.	The CCF/AZF responses to the API invoker/ROF/UE. The response includes indication whether revocation is successful, updated authorization information. If a token is issued, the token claims include the updated service operation/resource, and GPSI for RNAA.

### 6.10.3	Evaluation

This solution addresses the requirements of the Key Issue #1.3.

This solution provides an authorization procedure and a revocation procedure with finer granularity. Both procedures are reusing the mechanisms specified in TS 33.222 [3]. The granularity levels of authorization and/or revocation can be aligned to TS 33.222 [3].

***  END OF CHANGES  ***