3GPP TSG-SA3 Meeting #119	S3-245039

Orlando, US, 11 - 15 November 2024

# Source:	Samsung

# Title:	Discussion on security procedure on S&F operation for a split MME architecture

# Document for:	Endorsement

Agenda Item:	5.7

# 1	Decision/action requested

SA3 is kindly requested to endorse this contribution.

# 2	References

[1] 	3GPP TR 23.700-29 "Study on integration of satellite components in the 5G architecture; Phase 3"

[2] 	3GPP TR 33.700-29 "Study on security aspects of 5G satellite access in the 5G architecture; Phase 3"

# 3	Rationale

SA3 has concluded on the security aspect for S&F operation in SA3#118 meeting. Especially, SA3 agreed to enhance the security procedure to provide protection against DoS attacks before a security context is established for a split MME architecture. Considering the conclusion from TR 23.700-29 [1] for a split MME architecture, the attach procedure for a split MME architecture can be described as follows:

Figure 3-1: Attach procedure for a split MME architecture in S&F operation

To summarize the above figure:

PHASE 1: When only service link is available, the UE sends Attach Request and the satellite stores the Attach Request before sending the Attach Reject containing Re-attach information.

PHASE 2: When only feeder link is available, the satellite sends Auth Request containing IMSI to the ground network and receives Authentication Vector(s) from the ground network.

PHASE 3: When only service link is available, the UE performs the authentication procedure with the satellite which has Authentication Vector(s) for the UE.

As described in the key issue in TR 33.700-29 [2], current security procedure cannot prevent DoS attack on a satellite in PHASE 1. In PHASE 1, for example, an attacker can send a number of Attach Request messages with fake IMSIs to mount DoS attack. However, the satellite, which cannot verify whether the messages are from genuine UEs, needs to store the Attach Request messages in order to send the messages to the ground network in PHASE 2.

Observation 1: Current security procedure cannot prevent DoS attack on a satellite from a number of bogus Attach Requests with different IMSIs in PHASE 1.

In addition, according to the conclusion from TR 23.700-29 [1] for a split MME architecture, Attach Reject message in PHASE 1 includes a new information (e.g., wait timer, the list of satellite IDs) indicating the UE can re-attempt the attach in a next satellite pass. For example, to mount DoS attack on a UE, an attacker can send the Attach Reject message containing fake list of satellite IDs. This attack scenario is newly introduced from the conclusion in TR 23.700-29 [1] that the Attach Reject message includes re-attach information for S&F operation.

Observation 2: Current security procedure cannot prevent DoS attack on a UE from a fake Attach Reject message in PHASE 1.

Proposal 1: The key issue #1 in TR 33.700-29 [2] should capture the DoS attack on a UE, considering the conclusion from TR 23.700-29 [1].

Proposal 2: Enhancements to provide protection against DoS attacks before a security context is established should be able to prevent the DoS attacks on the UE and the satellite.

Some solutions in TR 33.700-29 [2] are addressing DoS attack(s) before security context is established (i.e., PHASE 1).

|  | Summary | Disadvantage | DoS attack on the satellite | DoS attack on the UE |
| --- | --- | --- | --- | --- |
| Sol#8 | ECCSI is used to protect Attach Request/Reject. | Credentials for ECCSI need to be provisioned to the UE and satellite. Legacy UE cannot generate/verify signature. | O | O |
| Sol#9 | PKI is used to protect Attach Reject. | Credentials for PKI need to be provisioned to the UE. Legacy UE cannot verify the signature. | X | O |
| Sol#21 | Puzzle is used to slow down the rate of Attach Requests. | DoS attack on the satellite cannot be fully prevented. Legacy UE cannot solve the puzzle. | △ | X |
| Sol#25 | Symmetric cryptography is used to protect Attach Request/Reject. | Credentials need to be provisioned to the UE and satellite. Legacy UE cannot generate/verify MAC. | O | O |
| Sol#27 | Group key is used to protect Attach Request/Reject | Credentials need to be provisioned to the UE and satellite. Legacy UE cannot generate/verify MIC. If one key is leaked, all of the UEs are affected. | O | O |
| Sol#32 | Shared secret is used to solve the puzzle. | Credentials need to be provisioned to the UE and satellite. Legacy UE cannot solve the puzzle. | O | X |
| Sol#33 | Integration of Sol#8 and #21 | Risk of DoS attack on the satellite still remains. This solution does not work for legacy UE. | △ | O |
| Sol#34 | Integration of Sol#9 and #21 | Risk of DoS attack on the satellite still remains. This solution does not work for legacy UE. | △ | O |
| Sol#35 | Integration of Sol#15 and #21 | Risk of DoS attack on the satellite still remains. DoS attack on the UE is not prevented. This solution does not work for legacy UE. | △ | X |

Observation 3: All of the solutions do not work for the legacy UEs.

Observation 4: Solution #8, Solution #25, and Solution #27 are addressing both DoS attack on the satellite and on the UE.

Observation 5: Regarding provisioning of the credentials, the conclusion from TR 23.700-29 [1] says that UE may have a USIM dedicated to the satellite network.

Observation 6: From the security point of view, Solution #27 has more disadvantage than Solution #8 and Solution # 25, due to the fact that group key is used.

Proposal 3: One of Solution #8 and Solution #25 can be used as a basis for the normative work to prevent DoS attacks before authentication is performed (i.e., PHASE 1).

# 4	Detailed proposal

Observation 1: Current security procedure cannot prevent DoS attack on a satellite from a number of bogus Attach Requests with different IMSIs in PHASE 1.

Observation 2: Current security procedure cannot prevent DoS attack on a UE from a fake Attach Reject message in PHASE 1.

Observation 3: All of the solutions do not work for the legacy UEs.

Observation 4: Solution #8, Solution #25, and Solution #27 are addressing both DoS attack on the satellite and on the UE.

Observation 5: Regarding provisioning of the credentials, the conclusion from TR 23.700-29 [1] says that UE may have a USIM dedicated to the satellite network.

Observation 6: From the security point of view, Solution #27 has more disadvantage than Solution #8 and Solution # 25, due to the fact that group key is used.

Proposal 1: Key issue #1 in TR 33.700-29 [2] should capture the DoS attack on a UE, considering the conclusion from TR 23.700-29 [1].

Proposal 2: Enhancements to provide protection against DoS attacks before a security context is established should be able to prevent the DoS attacks on the UE and the satellite.

Proposal 3: One of Solution #8 and Solution #25 can be used as a basis for the normative to prevent DoS attacks before authentication is performed (i.e., PHASE 1).