3GPP TSG-SA3 Meeting #119	S3-245095

Orlando, US, 11 -15 November 2024

# Source:	Ericsson

# Title:	New WID on MOBIKE for NTN 3GPP backhaul over the feeder link

# Document for:	Agreement

# Agenda Item:	6

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: MOBIKE for NTN 3GPP backhaul over the feeder link

Acronym: MOBIKE-5GNTN

Unique identifier:

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X |  |  |
| No | X | X |  | X |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

# 3	Justification

An NG-RAN consisting of a gNB embarked on an airborne or space-borne NTN vehicle communicates with the ground-based infrastructure via a feeder link of a ground-based NTN Gateway. The backhaul segment connecting the gNB on an NTN vehicle and the ground based SEG is carried over a feeder link. The orbiting satellite movement results into frequent switches of a feeder link from a source NTN Gateway to a target NTN Gateway. It may result into a change in IP address of a gNB. Consequently, that requires IKEv2 managing the change in outer IP addresses of IPsec tunnel and updating the security associations addresses.

A change request to TS 33.501 is required to introduce support for MOBIKE extension in IKEv2/IPsec between generic regenerative gNB and SEG for the optimization of IKEv2/IPsec secured backhaul over the feeder link.

# 4	Objective

Normative work to support for MOBIKE extension in IKEv2/IPsec to optimize 3GPP backhaul over the feeder link.

# 5	Expected Output and Time scale

TU estimate: 0.25

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| {Possible values: "TS" or "Internal TR" or "External TR". See Note 1} | {e.g. "22.XXX" or actual number if known} | {Title of the specification (as per TR 21.801 §6.1.1), to be aligned as much as possible with the WI/SI title} | {e.g. "TSG#87"} | {e.g. "TSG#89"} | {<FamilyName>, <GivenName>, <Company>, <email address>. See Note 2} |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 33.501 | Specify support for MOBIKE extension in IKEv2/IPsec for 3GPP backhaul between 5G NTN generic regenerative gNB and SEG. | TSG#106 | This TS covers Stage 2. There is no Stage 3 impact since existing interaction for IKEv2/IPsec and profile from TS 33.210 are reused. |

# 6	Work item Rapporteur(s)

Niraj Rathod, Niraj.rathod@ericsson.com

# 7	Work item leadership

SA3

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval}

Supporting IM name

Ericsson

Huawei?

Nokia?

CATT?

Interdigital?