3GPP TSG-SA3 Meeting #119	S3-245280

Orlando, Florida, 11th – 15th November 2024

# Source:	Johns Hopkins University APL, MITRE-FFRDC, US National Securiyt Agency

# Title:	Add standardized format for security event logs into conclusion of KI#1

# Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

Update KI#1 conclusion in TR 33.794 to add support a standardized format for security event logs.

# 2	References

[1]	3GPP TR 33.794 Study on enablers for Zero Trust Security

# 3	Rationale

Based on discussion of S3-245183, it was suggested to propose adding a point into the conclusion for KI#1 to include NF support for a standardized format of the data contained in the security event logs.

# 4	Detailed proposal

SA3 is kindly requested to approve the below change to TR 33.794.

********************** Start of 1st Change ****************************

# 8	Conclusions

Editor’s Note: This clause contains the agreed conclusions that will form the basis for any normative work.

## 8.1	Key Issue #1: Data exposure for security evaluation and monitoring

The security incidents or scenarios in SBA where data can be collected in the SBA layer includes

1)	authentication and authorization failure event;

2)	unexpected setup of TLS session and API invocation related to unauthorized reconnaissance;

3)	malformed message event;

4)	high service load;

5)	unexpected SBI call flows; and

6) unexpected use of APIs exposed by services in SBA layer

Editor’s Note: Further aspects if any related to KI#1 is FFS.

The key issue will be addressed by requirements for data collection to enable security evaluation and monitoring on the SBA layer. No new interface nor protocol will be specified as result of the work in this report.

The following requirements address KI#1 Data exposure for security evaluation and monitoring. The requirements also address the use cases for security evaluation and monitoring described in clause 5.1.

General requirements for security event logs:

1.	The NF supports the generation of security event logs.

Security event logs contain security relevant events data. Requirements related to security relevant events data for monitoring:

a.	The NF collects information on the SBA layer about received malformed messages that deviate from the 3GPP specified messages or are considered invalid according to the protocol specification and network state are to be logged. (Clause 5.1.1)

b.	The NF collects information about events involving a massive number of incoming messages on the SBA layer. (Clause 5.1.2).

c.	The NF collects information about authentication and authorization failure on the SBA layer. (Clause 5.1.3).

d.	The NF collects information about potential replay attacks on the SBA layer. (Clause 5.1.6).

X. The NF supports a set of data (e.g., information elements as applicable) for the security event logs.

NOTE: The format if any related to bullet X is out of scope of this document.

Editors note:	Bullet d) is FFS.

Editors note:	Additional details if any are FFS.

********************** End of 1st Change ****************************