3GPP TSG-SA3 Meeting #119 	S3-245306

# Orlando, US, 11 - 14 November 2024                                             revision of S3-244709

# Source:	Apple

# Title:	Solution#29 update to address ENs

# Document for:	Approval

Agenda Item:	5.6

# 1	Decision/action requested

Approve the pCR to TR 33.713

# References

- TR 33.713 v0.3.0

# 3	Rationale

This contribution proposes a new solution to address key issue#3 in TR 33.713.

# 4	Detailed proposal

*** Start of 1st Change ***

## 6.29	Solution #29: Privacy protection on AIoT device IDs

### 6.29.1	Introduction

This solution addresses key issue#3: “Mechanisms for mitigating privacy threats (described above) by identifying, linking, and tracking the identifiers of AIoT Device(s) shall be supported.”

### 6.29.2	Details

Preassumption: the AIoT device has one ID preconfigured by the manufacture (Application Function). Each device is configured with a root key, named K. K is unique for every AIoT device.

The device shall also maintain an Index to indicate to the network that how to map the HASH value to the HASH table. For example, when the Index is n, it indicates this is the n-th calculation of the HASH value of the device ID. The Index shall be sent uplink together with the HASH (device ID||Index).

Step 0a. the device is configured with one Device ID in the manufacturing time. Every ID is configured corresponding

to an Index. The initial Index value is 0.

Editor’s Notes: How to address the synchronizaiton issue, e.g. when one AIoT device is connected to multi Readers, how to sync among those Readers on the HASH values of device IDs, is FFS.

Editor’s Notes: It is FFS how the procedure is integrated into inventory procedure, e.g. how the Reader report the device ID to the AF, when the inventory procedure is triggered.

Step 0b. the Application Function sends the Device IDs and Indexes to the AIoTF through NEF.

Step 0c. AIoTF sends the corresponding Device IDs to each Reader based on distribution policy from Application Function or local policy from MNOs.

Step 0d. Reader stores the Devices IDs and the corresponding Indexes under this Reader.

Step 1. Reader sends the paging message to the AIoT devices. Reader sends E(ID1-0 ), E(ID2-0 ), … in the paging message. The E() Function is HMAC, the key used is the root key K.

NOTE 1: According to RAN2 agreement in #116, the paging message may contain one ID (one Device ID or one group ID) or more (multiple IDs FFS in RAN2) IDs. The procedure may be updated based on RAN2 progress.

NOTE 2: IDm-n indicates the n-th ID calculation using HASH for Device m. ID1-0 indicates the original device ID. ID1-1 = HASH (ID1-0  ||(Index = 1))

Step 2: AIoT Device1 verifies the IDs being paged, if verification is successful and one of the device IDs maps its ID, AIoT Device 1 confirms it was paged. Then Device 1 replies with ID1-1||Index to the Reader. Index should be 1 in this message, meaning this is the first time the device ID1 was HASHed.

NOTE 3: The IDm-n  sent uplink doesn’t have to be sequencial,. For example, the device could send the ID1-1||(Index=1), it can then send ID1-3||(Index=3) after ID1-1||(Index=1).

Step 3: Reader compares the ID1-1 with its HASH table, then confirm this ID1-1 is in its data base.

Step 4: Reader echoes back the ID1-1 to device, following RAN2 procedure.

Step 5: Device checks ID1-1 is correct, use ID1-2 in next message, in which ID1-2 = HASH (ID1-1 ||(Index =2))

NOTE 4: Reader shall use unused ID for each device for the next paging if there is any.

### 6.29.3	Evaluation

The impact on the AIoT device and the network are as following:

The AIoT device needs to support to:

- maintain the root key,

- decrypt the encrypted device ID using the root key,

- calculate the HASH (device ID||Index)

The network needs to support to:

- store the keys for each device ID,

- paging using encrypted device ID

- search the HASH table to verify the device ID sent uplink

Editor’s Note: Further evaluation is FFS.

*** End of 2nd Change ***