3GPP TSG-SA3 Meeting #119	S3-244877

Orlando, US  11 - 15 November 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2074 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to MPQUIC TLS Annex |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-11-4 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In R18 ATSSS scenario, existing air interface protection and IPsec between RAN and UPF provide protection for data transmission. MPQUIC functionality uses QUIC protocol, which integrates TLS and provides additional security feature on the steering function layer.Other high layer steering functionalities (e.g. MPTCP, ATSSS-LL) than MPQUIC do not provide such additional security feature. From this point of view, this high-level security is not essential, and the consumption of this function can be minimized. Based on the previous duscussions, current version of Annex AA only allows the UE to use X.509 certificate to authenticate the UPF, while TLS also support other options. We should not limit the use of various approaches for building security, especially when other approaches have additional benefits, such as reducing the cost of communication and computation, and making deployment more flexible. It is worth mentioning that the exisitng specification exclude the hard-coded pre-shared key option, hard-coded pre-shared key means that the pre-shared key is directly written into the code and cannot be changed unless the software is upgraded. Pre-shared key can be deployed by other means like pre-configuration, and is align with existing specification. Specfically, pre-configured pre-shared keys can be changed at any time through configuration means. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to add pre-shared key authentication option for the use of TLS authentication when MPQUIC is utilized as the multipath protocol for ATSSS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Restrictions on deployment mode and unnecessary resource cost for some scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex AA |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** START OF 1st CHANGES ****

Annex AA (normative): 
Security aspects of the Access Traffic Steering, Switching and Splitting (ATSSS)

AA.1	General

This Annex specifies the Security aspects of the Access Traffic Steering, Switching and Splitting (ATSSS). The ATSSS feature is described in 3GPP TS 23.501 [2].

AA.2	Server authentication for MPQUIC in ATSSS

When multipath QUIC (MPQUIC) [115, 116, 117] steering functionality is used for ATSSS, RFC 9001 [x2] mandates the use of TLS to secure QUIC.

NOTE: TLS server authentication based on X.509 certificates can be selected for authentication of the MPQUIC Proxy in the UPF, which is to be performed by the MPQUIC Client in the UE. Self-signed end entity certificates and hard-coded pre-shared keys need to be avoided. Establishing the trust anchor in the UE is left to deployment. If no trust anchor is provisioned in the UE, the entire certificate chain including the root CA certificate needs to be provided to the UE by the UPF during TLS handshake, and the root CA certificate is implicitly trusted.

NOTE: TLS authentication based on pre-shared keys can also be selected for authentication of the MPQUIC Proxy in the UPF, which is to be performed by the MPQUIC Client in the UE. The pre-shared key can be pre-configured by out of band means or derived from 5G security context, hard-coded pre-shared keys need to be avoided. The pre-shared keys pre-configured for UPFs in the same PLMN can be the same or different, which is determined by the operator.

**** END OF CHANGES ****