3GPP TSG-SA3 Meeting #119	S3-24XXXX

Orlando, US  11 – 15 November 2024													   revision of

# Source:	Nokia

# Title:	Update to KI2 Solution 12 on security method retrieval in CAPIF interconnect

# Document for:	Approval, Information, Discussion

Agenda Item:	5.19

# 1	Decision/action requested

Solution on security method retrieval in CAPIF interconnect is updated.

# 2	References

[1]	3GPP TS 23.700-22

[2]	3GPP TS 33.700-22

# 3	Rationale

KI#2 solution on CAPIF interconnect needed. Aspect of security method retrieval.

Evaluation of the solution has been added.

# 4	Detailed proposal

**********   START OF CHANGES

### 6.12.3	Evaluation

TBD

The solution address KI#2 by requesting the security method to be used with AEF directly to its responsible CCF (CFF-A in figure).

The solution affects CCFs functionalities and their inter-communication, by allowing one CCF to forward the security method request to a second CCF.

From key issue KI#2 CAPIF interconnect the aspect security method retrieval to allow for authentication and authorization of the API invoker to the AEF is addressed.

Additional communication between CCFs of different domains is needed.

**********   END OF CHANGES