3GPP TSG-SA3 Meeting #119	S3-245184

Orlando, USA 11-15 November 2024			revision of S3-244723, merger of S3-244610, S3-245081

# Source:	US National Security Agency, OTD_US, Johns Hopkins University APL, MITRE-FFRDC, Ericsson, Motorola Mobility, Nokia

# Title:	General Recommendations for Conclusion 2 Policy Enforcement

# Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

It is proposed to approve the proposed changes to TR 33.794 clause 8.2 conclusions for KI 2

# 2	References

[8] 	NIST Special Publication 800-207: "Zero Trust Architecture".

# 3	Rationale

This submission provides additions to conclusion 2 including some general recommendations.

# 4	Detailed proposal

*****Start Changes*****

## 8.2	Key Issue #2: Security mechanisms for policy enforcement at the 5G SBA

No normative work is needed for KI#2.

For security policy enforcement and improved access control decisions based on security evaluation and monitoring results following aspects are described as follows:

- An NF acting as a Policy Enforcement Point (PEP) in SBA performs actions based on security evaluation results/outcome (e.g., in case of NF being identified/suspected to be compromised) during NF service requests (e.g., NF service discovery, access token requests, NF service update) and there may be more than one PEP within the SBA.

- NF (i.e., service producer or service consumer) acts as PEP (i.e., in case of deployment A when no NRF and SCP involved as well as in general case of SBA service request/response as applicable)

- NRF acts as PEP (i.e., in case of deployment B where NRF is involved).

- NRF and SCP acts as PEPs (i.e., in case of deployment C first NRF is involved in Service discovery and then SCP is involved during service request).

- SCP acts as a PEP (i.e., in case of deployment D).

NOTE 1: Communication models for NF/NF services interaction are described in TS 23.501 [18].

- If SBA NF performs PEP functions (e.g. as described in NIST [8]) then following actions are needed:

- The PDP instructs the PEP i.e., actions to the NF functioning as a PEP.

NOTE 2: PDP decides on the security issues/incidents and how the related responses were decided is left to MNO’s implementation. It is upto the MNO to decide the security issues and the corresponding responses.

- PEP takes actions according to (2.a).

*****End Changes*****