3GPP TSG-SA3 Meeting #119	S3-245313

# Orlando, US, 11 - 15 Nov 2024

# Source: 	OPPO, Xidian

# Title: 	Remove ENs and add evaluation in Solution #17

# Document for: 	Approval

Agenda Item: 	5.9

1	Decision/action requested

Approve the pCR to TR 33.713

2	References

N/A

3	Rationale

This contribution proposes to remove ENs and add evaluation in Solution #17 of TR 33.713. This pCR includes the following updates:

- Remove the following Editor’s Notes:

Editor’s Note: How the AIoT device maintans the counter is FFS.

- Add evaluation for this solution.

4	Detailed proposal

SA3 is requested to approve the following pCR.

*** Start of Change ***

6.17	Solution #17: Disabling operation procedure for AIoT services

6.17.1	Introduction

The solution addresses the security requirement of KI#1: Protection for disabling device operation. According to TS 22.369 [2], the network operator shall provide a suitable mechanism to temporarily/permanently disable the capability of an Ambient IoT device or a group of Ambient IoT devices to transmit RF signals. The solution assumes the network operator is responsible for managing the AIoT device using the temporarily/permanently disable operation.

Specifically, the AIoT NF sends the AIoT Disable Request to the AIoT device. The message includes the disable security parameter, which is calculated by the shared key between AIoT devices and AIoT NF. The AIoT device calculates and verifies the disable security parameter using the same shared key to decide whether to perform the disable operation.

6.17.2	Solution details

Depicted in Figure 6.17.2-1 is the disable operation procedure for AIoT Services.

Figure 6.17.2-1: Disable operation procedure for AIoT service

- The AF sends the AIoT Disable Request to the AIoT NF via the NEF, including the AIoT device filter information, and disable type (i.e., temporarily/permanently disable).

- The AIoT NF sends the AIoT Disable Trigger to the RAN/UE Reader, including the AIoT device filter information.

The AIoT device filter information is used to page an AIoT device or a group of AIoT device, which could be the AIoT device ID, the group ID, or the partial ID of AIoT device.

- The RAN/UE Reader sends the AIoT Paging Request to the AIoT device, including the AIoT device filter information.

- The AIoT device checks whether the ID matches according to the AIoT device filter information.

- The AIoT device sends the AIoT Paging Response to the RAN/UE Reader, including the AIoT device ID.

- The RAN/UE Reader sends the AIoT Disable Trigger Response to the AIoT NF, including the AIoT device ID.

- The AIoT NF uses f function to calculate the disable security parameter with the input parameters key K, AIoT ID, disable type and counter, i.e., f (K, AIoT ID, disable type, counter). The initial value of the counter is set to zero and AIoT NF increases the counter by one for each subsequent disable operation. When the counter overflows, the AIoT NF resets the counter to zero.

NOTE: The f functions can be HMACfunctions, f1-f5 functions, AES functions, etc.

NOTE: The key K can be the root key or its derived key of the AIoT device.

- The AIoT NF sends the AIoT Disable Request to the RAN/UE Reader. The message contains the AIoT ID, disable security parameter, disable type and counter.

- The RAN/UE Reader forwards the AIoT Disable Request to the AIoT device.

- The AIoT device is required to store the counter. When the received counter in the AIoT Disable Request is less than the local counter, the device refuses the Request, otherwise the AIoT device uses the same shared key to verify the disable security parameters.

- (a) The AIoT device performs the disable operation according to the disable type, if the calculated disable security parameter matches the received disable security parameter.

(b) The AIoT device sends the AIoT Disable Response to the RAN/UE Reader before disabling the RF capability, if the AF requires a feedback.

- The RAN/UE Reader forwards the AIoT Disable Response to the AIoT NF.

- The AIoT NF sends the AIoT Disable Response to the AF via the NEF.

6.17.3	Evaluation

This solution fulfils the security requirements in key issue #1.

This solution addresses the security of the Command Operation (i.e., manipulation) procedure by requiring the AIoT device to maintain the counter.

This solution requires the AIoT device and AIoT NF to be provisioned with a shared key to calculate the disable security parameter.

Editor’s Note: Further evaluation is FFS.

*** End of Change ***