3GPP TSG-SA WG3 Meeting #55 	S3-090798

- Shanghai, China, 11-15 May 2009

| CR-Form-v9.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.820 | CR | 0020 |  rev | - |  Current version: | 8.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Adding the other scenario in the system architecture of HNB and HeNB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | China mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code:  |  |  |  |  |  |  |  | Date:  |  | 29/04/2009 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) |  |
| Reason for change:  |  | Referring to description of system architecture of HNB and HeNB, TR33.820 v.8.0.0 has been considering the backhaul between H(e)NB and SeGW may be insecure. However, security tunnel is established between H(e)NB and SeGW without considering whether the backhaul link is secure or not. It is necessary to add another scenario in the system architecture of H(e)NB when the backhaul link is secure enough. If the backhaul link between H(e)NB and SeGW is secure enough, the security tunnel is not necessary to be made. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | To modify system architecture figures about H(e)NB and to add a note to make security tunnel be optional in the scenario that the backhaul between HNB and SeGW is secure enough. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Incomplete description on H(e)NB system architecture |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 4.2, 4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

************************************ start of 1st change ************************************

## 4.2	System architecture of HNB

UE

HNB

SeGW

Unsecure/secure link

Operator’s core network

OAM

Figure 1: System Architecture of HNB

Description of proposed system architecture:

- Air interface between UE and HNB should be backwards compatible air interface in UTRAN;

- HNB access operator’s core network via a Security Gateway. The backhaul between HNB and SeGW may be insecure.

- Security Gateway represent operator’s core network to perform mutual authentication with HNB. Mutual authentication may need support of authentication server or PKI.

- Security tunnel is established between HNB and Security Gateway to protect information transmitted in backhaul link.

Note: Security tunnel shall be optional when the backhaul between HNB and SeGW is secure enough.

- Secure communication is required to Operation, Administration and Maintenance (OAM). This becomes even more important if OAM is placed outside the operator’s network.

Editor’s Note: The security implications of collapsing certain Core networks related functionality (e.g. SGSN or GGSN) )in the HNB should be studied

NOTE: There may be a Home Gateway in the architecture at the customer premise. If such a Home Gateway is a physically and logically separate entity than the HNB, such a Home Gateway should not be present in the architecture since the security of the HNB should not rely on the security of the Home Gateway. However, if such a Home Gateway is physically or logically integrated with a HNB, it should be studied if security aspects (e.g. device security) of the Home Gateway may impact that of the HNB. In addition, the existence of any Home Gateway (integrated or separated) may imply restriction on the selection of backhaul security solutions, e.g. to allow NAT traversal.

## 4.3	System architecture of HeNB

UE

HeNB

SeGW

Unsecure/secure link

Operator’s core network

OAM

Figure 2: System Architecture of HeNB

Description of proposed system architecture:

- Air interface between UE and HeNB should be backwards compatible with air interface in E-UTRAN;

- HeNB access operator’s core network via a Security Gateway. The backhaul between HeNB and SeGW may be insecure.

- Security Gateway represent operator’s core network to perform mutual authentication with HeNB. Mutual authentication may need support of authentication server or PKI.

- Security tunnel is established between HeNB and Security Gateway to protect information transmitted in backhaul link.

Note: Security tunnel shall be optional when the backhaul between HeNB and SeGW is secure enough.

- Secure communication is required to Operation, Administration and Maintenance (OAM). This becomes even more important if OAM is placed outside the operator’s network.

Editor’s Note: The security implications of collapsing certain Core networks related functionality (e.g. Serving GW) )in the HeNB should be studied

NOTE: There may be a Home Gateway in the architecture at the customer premise. If such a Home Gateway is a physically and logically separate entity than the HNB, such a Home Gateway should not be present in the architecture since the security of the HNB should not rely on the security of the Home Gateway. However, if such a Home Gateway is physically or logically integrated with a HNB, it should be studied if security aspects (e.g. device security) of the Home Gateway may impact that of the HNB. In addition, the existence of any Home Gateway (integrated or separated) may imply restriction on the selection of backhaul security solutions, e.g. to allow NAT traversal.

************************************ end of 1st change ************************************