3GPP TSG-SA WG3 Meeting #55	S3-09xxxx

- Shanghai, China, 11-15 May 2009

| CR-Form-v9.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.109 | CR |  |  rev | - |  Current version: | 8.1.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add MSISDN in Zn |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code:  | GBA |  |  |  |  |  |  | Date:  |  | 12/5/2009 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) |  |
| Reason for change:  |  | For a NAF deployed in an R6 before network, especially an operator’s own NAF, it is necessary to generate billing record according to users’ MSISDN. However, if a GBA-based authentication mechanism is used between user and the NAF, for example, HTTP Digest in MBMS service, the user is identified by B-TID. So NAF should be able to translate B-TID into MSISDN when it needs to generate billing record according to the user’s subscription request after authenticating the user successfully. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | It aims to add the requirement in TS 33.220 that BSF can send MSISDN to NAF through Zn reference point for the network that HLR is used. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | In lots of services, subscribers are usually identified by MSISDN in service plane, especially in billing systems. But In 33.220, NAF cannot retrieve MSISDN from BSF. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

*************Begin of 1st change*************

## 5.2	Protocol Zn between NAF and BSF based on Diameter

The requirements for Zn interface are defined in 3GPP TS 33.220 [5].

The protocol Zn retrieves the key material and possibly user security settings data by NAF from BSF.  After UE is authenticated with the BSF, every time the UE wants to interact with an NAF the following steps are executed as depicted in Figure 5.3. The basic procedure is:

…

Step 3

After that the BSF shall send a Bootstrapping-Info-Answer message (BIA) back to the NAF.

< Boostrapping-Info-Answer> ::= < Diameter Header: 310, PXY, 16777220 >
< Session-Id >
{ Vendor-Specific-Application-Id }
[ Result-Code ]	
[ Experimental-Result]
{ Origin-Host }	; Address of BSF
{ Origin-Realm }	; Realm of BSF
[ User-Name ]	; IMPI or MSISDN if HLR is used
[ ME-Key-Material ]	; Required
[ UICC-Key-Material ]	; Conditional
[ Key-ExpiryTime ]	; Time of expiry
[ BootstrapInfoCreationTime ]	; Bootstrapinfo creation time
[ GBA-UserSecSettings ]	; Selected USSs
[ GBA-Type ]	; GBA type used in bootstrapping
*[ AVP ]
*[ Proxy-Info ]
*[ Route-Record ]

The BSF may or may not send  the User-name AVP (IMPI) according its configuration.

*************End of 1st change*************