3GPP TSG-SA WG3 Meeting #55	S3-09xxxx

- Shanghai, China, 11-15 May 2009

| CR-Form-v9.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.223 | CR |  |  rev | - |  Current version: | 8.3.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | GBA_U should be verified in registration procedure in GBA Push in case of HLR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code:  | GBA Push |  |  |  |  |  |  | Date:  |  | 12/5/2009 |
| Category:  | C |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) |  |
| Reason for change:  |  | In GBA Push, indicator for use of GBA_ME or GBA_U should be included in GPI request from NAF to BSF. If GBA_U is requested the BSF checks if GBA_U is supported in GUSS in case HSS is utilized. But if the network utilizes a HLR instead of HSS, then the BSF request only AV from HLR. If the USIM card is not a GBA_U awareness card, some errors will be happened. Since the U/M is request from NAF in the GPI request, it’s necessary for NAF to know the GBA capability of USIM. In the registration procedure, it is proposed to verify the GBA capability of USIM. For example, in the registration, NAF should know the GBA capability of UE by checking the USIM or retrieving from operator, just like the card management system. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Add NOTE in the annex B that the USIM should be checked whether GBA_U is supported in the registration procedure, especially HLR is utilized instead of HSS in GBA Push. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | If the network utilizes a HLR instead of HSS, then the BSF request only AV from HLR. If the USIM card is not a GBA_U awareness card, some errors will be happened. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

**** Start of 1st change *****

Annex B (informative):
GBA-Push UE registration procedure

To be able to use GBA Push based services the user and the service provider need to share information. This is done in a registration procedure. The registration procedure could be explicit and involve the user or it could be automatic relying on user information provided by the user's operator. If the registration is initiated by the operator, the operator will have access to all needed registration information. When a user registers with a public identity this might not be the case, especially if the NAF is a third party service provider. One way of alleviating the problem would be to have users perform the registration over an authenticated/secured connection established with normal GBA. Then the BSF could provide the NAF with all needed information. Note however that this functionality is not standardized and that all needed information might not be available over the currently standardized interfaces.

NOTE: If GBA_U should be used by NAF, it’s necessary for NAF to check whether the GBA_U is supported by USIM in the registration procedure, especially HLR is utilized instead of HSS.

At the registration the Push NAF needs to record the user identity (UE_Id), a push delivery method and the associated transport address (UE_Trp). The user identity may be either a public identity or a private identity.

**** End of 1st change *****