3GPP TSG-SA WG3 Meeting #55	S3-09xxxx

- Shanghai, China, 11-15 May 2009

| CR-Form-v9.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.220 | CR |  |  rev | - |  Current version: | 8.6.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add MSISDN in Zn |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code:  | GBA |  |  |  |  |  |  | Date:  |  | 12/5/2009 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) |  |
| Reason for change:  |  | For a NAF deployed in an R6 before network, especially an operator’s own NAF, it is necessary to generate billing record according to users’ MSISDN. However, if a GBA-based authentication mechanism is used between user and the NAF, for example, HTTP Digest in MBMS service, the user is identified by B-TID. So NAF should be able to translate B-TID into MSISDN when it needs to generate billing record according to the user’s subscription request after authenticating the user successfully. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | It aims to add the requirement in TS 33.220 that BSF can send MSISDN to NAF through Zn reference point for the network that HLR is used. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | In lots of services, subscribers are usually identified by MSISDN in service plane, especially in billing systems. But In 33.220, NAF cannot retrieve MSISDN from BSF. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

*************Begin of 1st change*************

### 4.4.6	Requirements on reference point Zn

The requirements for reference point Zn are:

-	mutual authentication, confidentiality and integrity shall be provided;

-	If the BSF and the NAF are located within the same operator's network, the DIAMETER based Zn reference point shall be secured according to NDS/IP [13];

-	If the BSF and the NAF are located in different operators' networks, the DIAMETER based Zn' reference point between the Zn-Proxy and the BSF shall be secured using TLS as specified in RFC 2246 [28];

NOTE 1:	Annex E specifies the TLS profile that shall be applied.

-	An HTTP based Zn/Zn' reference point shall be secured using TLS as specified in RFC 2246 [28];

NOTE 1b:		Annex E specifies the TLS profile that shall be applied.

-	The BSF shall verify that the requesting NAF is authorised to obtain the key material or the key material and the requested USS;

-	The NAF shall be able to send a key material request to the BSF, containing NAF's public hostname used by the UE's corresponding request. The BSF shall be able to verify that a NAF is authorized to use this hostname, i.e. the FQDN used by UE when it contacts the NAF;

-	The BSF shall be able to send the requested key material to the NAF;

-	The NAF shall be able to get a selected set of application-specific USSs from the BSF, depending on the policy of the BSF and the application indicated in the request from the NAF over Zn;

-	The NAF shall be able to indicate to the BSF the single application or several applications it requires USSs for;

NOTE 2:	If some application needs only a subset of an application-specific USS, e.g. only one IMPU, the NAF selects this subset from the complete set of USS sent from BSF.

-	The BSF shall be able to be configured on a per NAF or per application basis

-	whether private subscriber identity, i.e. IMPI, may be sent to the NAF;

-	whether a particular USS may be sent to a NAF;

NOTE 3:	 Privacy issues need be considered when determining which user identifier is sent to the NAF. If service continuity is desired, then the BSF can be configured to send the IMPI. If HLR is utilized instead of HSS,  BSF can be configured to send MSISDN (but then there is no user anonymity). If the BSF does not send the IMPI or IMPU / pseudonym in the USS, then the user remains anonymous towards the NAF; or more precisely, the B-TID functions as a temporary user identifier. This can cause that the NAF cannot provide a continuous service, since a user identity is needed in the NAF to ensure that the NAF is able to update keys for a Ua session when the UE has bootstrapped and contacts the NAF with a new B-TID. If user privacy is desired, the NAF can requests a USS and the BSF is configured to send a user pseudonym in the USS, but not the IMPI.

*************End of 1st change*************