# 3GPP TSG-SA3 (Security)	S3-111228

# SA3#65, 7-11 November 2011, San Diego, USA	revision of S3-111059

# Source: 	Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, AT&T, British Telecom, China Mobile, Ericsson, ST-Ericsson, Nokia Corporation, Nokia Siemens Networks, Qualcomm, Rogers Wireless

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.

# Title * : GBA extensions for re-use of SIP Digest credentials

## Acronym * : GBA-ext

## Unique identifier *tba

## 1	3GPP Work Area *

|  | Radio Access |
| --- | --- |
| x | Core Network |
| x | Services |

## 2	Classification of WI and linked work items

### 2.0	Primary classification *

This work item is a … *

|  | Study Item (go to 2.1) |
| --- | --- |
|  | Feature (go to 2.2) |
| X | Building Block (go to 2.3) |
|  | Work Task (go to 2.4) |

### 2.1	Study Item

| Related Work Item(s) (if any] |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Go to §3.

### 2.2	Feature

| Related Study Item or Feature (if any) * |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Go to §3.

### 2.3	Building Block

| Parent Feature (or Study Item) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |
| 520030 | Security small enhancements (Sec11) | Sec11 is the umbrella feature for Release 11 security work. |

This work item is … *

|  | Stage 1 (go to 2.3.1) |
| --- | --- |
| X | Stage 2 (go to 2.3.2) |
|  | Stage 3 (go to 2.3.3) |
|  | Test spec (go to 2.3.4) |
|  | Other (go to 2.3.5) |

#### 2.3.1		Stage 1

| Source of external requirements (if any) * |  |  |
| --- | --- | --- |
| Organization | Document | Remarks |

Go to §3.

#### 2.3.2		Stage 2  *

| Corresponding stage 1 work item |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Other source of stage 1 information |  |  |
| --- | --- | --- |
| TS or CR(s) | Clause | Remarks |

If no identified source of stage 1 information, justify: GBA originated in SA3 without a stage 1 work item as GBA is a generic security tool. This WI extends GBA. *

Go to §3.

#### 2.3.3		Stage 3 *

| Corresponding stage 2 work item (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Else, corresponding stage 1 work item |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Other justification |  |  |
| --- | --- | --- |
| TS or CR(s) Or external document | Clause | Remarks |

If no identified source of stage 2 information, justify: *

Go to §3.

#### 2.3.4		Test spec *

| Related Work Item(s) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

Go to §3.

#### 2.3.5		Other *

| Related Work Item(s) |  |  |  |
| --- | --- | --- | --- |
| Unique ID | Title | Nature of relationship | TS / TR |

Go to §3.

### 2.4		Work task *

| Parent Building Block |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

## 3	Justification *

This Work Item defines an extension of the Generic Bootstrapping Architecture (GBA) to allow the re-use of bootstrapping procedure based on a particular type of non-UICC credentials, namely SIP Digest credentials. GBA is a generic security tool or security enabler which can be used for a variety of purposes, e.g. bootstrapping security for accessing XCAP servers for user self-management over the 3GPP-defined Ut interface.

The Generic Bootstrapping Architecture (GBA), as defined in 3GPP TS 33.220, provides a bootstrapping mechanism, but it is limited to UICC-based credentials. This means that SIP Digest credentials cannot benefit from GBA. SIP Digest credentials are user names and shared secrets, or passwords, which are stored in the HSS, and either in the terminal or held by the user.

Extending the GBA bootstrapping procedure to re-use SIP Digest credentials for bootstrapping specific security  would bring the benefit of not requiring a separate bootstrapping security infrastructure. In this way, using SIP Digest credentials as part of GBA bootstrapping procedure would reduce the cost of the security infrastructure for the operator whose subscribers use SIP Digest credentials, but do not have a UICC available.

## 4	Objective *

The objective of this work item is to specify Stage 2 requirements and mechanisms for allowing SIP Digest credentials to be re-used by an extension of the GBA bootstrapping procedures.  Especially the following will be done:

-	specify needed extensions to Ub reference point to support re-use of SIP digest credentials for GBA

-	specify needed extensions to Zh reference point to support re-use of SIP digest credentials for GBA

-	specify needed extensions to Zn reference point to support re-use of SIP digest credentials for GBA, if any are identified

Currently no extensions are envisaged for Ua reference point since Ua reference point is application specific.

The use of the extension of GBA that is the subject of this WID shall be restricted to environments where the UICC is not available to subscribers.

## 5	Service Aspects

The envisaged extension of GBA will permit devices with SIP Digest credentials, but without UICC to bootstrap security for manifold services, e.g. secure access to XCAP servers for user self-management, without the need for the deployment of a separate security infrastructure. This would ease the introduction of new services for the operators whose subscribers use SIPbased credentials.

## 6	MMI-Aspects

The MMI will be involved if the SIP Digest password is held by the user. Alternatively, the SIP Digest password can be securely stored in the terminal. The envisaged GBA extension could lead to an improved user experience on the MMI.

## 7	Charging Aspects

None.

## 8	Security Aspects

This is a security work item.

## 9	Impacts *

| Affects: | UICC apps | ME | AN | CN | Others |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  |  |
| Don't know |  |  |  |  |  |

## 10	Expected Output and Time scale *

| New specifications * [If Study Item, one TR is anticipated] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spec No. | Title |  | Prime rsp. WG | 2ndary rsp. WG(s) | Presented for information at plenary# |  | Approved at plenary# |  | Comments |
| Affected existing specifications * [None in the case of Study Items] |  |  |  |  |  |  |  |  |  |
| Spec No. | CR | Subject |  |  |  | Approved at plenary# |  | Comments |  |
| TS 33.220 |  | Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA) |  |  |  | SA#55 (March 2012) |  | Addition of GBA extension e.g. in a normative Annex |  |
| TS 29.109 |  | Generic Authentication Architecture (GAA); Zh and Zn Interfaces based on the Diameter protocol; Stage 3 |  |  |  | SA#57 (Sept 2012) |  | Stage 3 specifications for GBA |  |
| TS 24.109 |  | Bootstrapping interface (Ub) Protocol details |  |  |  | SA#57 (Sept 2012) |  | Stage 3 specifications for GBA |  |

## 11		Work item rapporteur(s) *

Silke Holtmanns (Nokia Corporation)

## 12		Work item leadership *

SA3

## 13		Supporting Individual Members *

Supporting IM name

Alcatel-Lucent

Alcatel-Lucent Shanghai Bell

AT&T

British Telecom

China Mobile

Ericsson

ST-Ericsson

Nokia Corporation

Nokia Siemens Networks

Qualcomm

Rogers Wireless

form change history:

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"