3GPP TSG SA WG3 (Security) Meeting #74	S3-140184

20 - 24 January 2014;Taipei, Taiwan (province of China)

Source:	China mobile, China Unicom, TeliaSonera

Title:	Assets and external interfaces of MME /comment on contribution from Alcatel-Lucent

Document for:	Approval

Agenda Item:	7.15

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution: This contribution analizes the assets and the external interfaces of MME.

# Introduction

We would like to kick off the inputs to the MME SAS. In SECAM study, it was pointed out that the first step of writing SPD is to list the critical assets of the network product class and identify the external interfaces of this class. We basically list the assets and the external interfaces of MME in this document as the target of the threat analysis, and help to forward the security threats and requirement discussion.

# Analysis

- Through the functionality analysis and a basic summary of MME, we can find the assets of MME to be protected are:Access-account and control data, credentials, and log data.

- Interface Configuration data: MME’s IP address, ports, VPN ID etc.

- OS

- Application(s)

- Mobility Management data: subscriber’s identities (eg. IMSI), subscriber keys (i.e KNASenc, KNASint, NH), authentication parameters, address of serving eNB, APN name, data related to mobility management like UE status, UE’s IP address,etc., session management like PDN type, QoS and so on, or node selection and routing selection, e.g. IP address of UE related S/P-GW, selected routing connection based on UE’s identity, etc.

- Sufficient processing capacity: that processing powers are not consumed close to limits.

- Hardware: mainframe, board, power supply unit etc.

The external interfaces of MME to be protected are:

- Console interface, for local access: local interface on MME

- O&M interface, for remote access: interface between MME and O&M system

- S1-MME: between MME and eNB

- S3: between MME and S4 SGSN

- S10: between MME and MME

- S11: between MME and S-GW

- Gn: between MME and Gn/Gp SGSN

- S6a: between MME and HSS

- SGs: between MME and MSC

# Proposal

It is proposed to include the analysis below to TR 33.8xx.

# pCR

***	BEGIN CHANGES	***

# X Security Problem Definition

## X.1 Assets and external interfaces of MME

The assets of MME to be protected are:

- Access-account and control data, credentials, and log data.

- Interface Configuration data: MME’s IP address, ports, VPN ID etc.

- OS

- Application(s)

- Mobility Management data: subscriber’s identities (eg. IMSI), subscriber keys (i.e KNASenc, KNASint, NH), authentication parameters, address of serving eNB, APN name, data related to mobility management like UE status, UE’s IP address,etc., session management like PDN type, QoS and so on, or node selection and routing selection, e.g. IP address of UE related S/P-GW, selected routing connection based on UE’s identity, etc.

- Sufficient processing capacity: that processing powers are not consumed close to limits.

- Hardware: mainframe, board, power supply unit etc.

The external interfaces of MME to be protected are:

- Console interface, for local access: local interface on MME

- O&M interface, for remote access: interface between MME and O&M system

- S1-MME: between MME and eNB

- S3: between MME and S4 SGSN

- S10: between MME and MME

- S11: between MME and S-GW

- Gn: between MME and Gn/Gp SGSN

- S6a: between MME and HSS

- SGs: between MME and MSC

***	END OF CHANGES	***