3GPP TSG SA WG3 (Security) Meeting #74	S3-140243

January 20 – 24, 2014, Taipei, China

Source:	Huawei, HiSilicon

Title:	OM user authorization / Comment contribution by Alcatel-Lucent replaces S3-140243

Document for:	Approval

Agenda Item:	7.15

Work Item / Release:	R-13

Abstract of the contribution: This contribution provides a description of the requirement for OM user privilege authorization in MME.

# Introduction

This contribution provides a description of the requirement for OM user privilege authorization in MME. The privilege authorization is based on the combination of users, the management object and the operation.

# Analysis

When a user does the management task on a network and a network element, privileges must be under control. A user should be restricted to do the limited management operation on the limited objects.

If the user authorization rule is violated, the system can experience the following:

- Information disclosure

- Network functionality broken

In management systems, users having the same privileges are classified into groups. All users in the same group have the same privileges on the network O&M. The privileges are defined on the basis of doing the specification operation on the specification objects. The operations refer to the action of the commands, like “set”, “display”, etc. The operations also define the domain of FACAPS. For example, a user/user group is authorized to check the alarms but cannot configure the network.  The objects refer to the physical or the logical objects to be managed.

# Proposal

The proposed prifilege management requirement is shown in following figures.

Figure 1 OM Privilege Management Requirement Model: the Management Object

In Figure 1, the user groups are assigned the privilege to operate diferent MOs (management objects).

Figure 2 OM Privilege Management Requirement Model: Command Group

In Figure 2, the user groups are assigned the privilege to operate diferent MOs (management objects).

# pCR

***	BEGIN CHANGES	***

# X O&M privilege management requirements on MME Management and Maintenance

A Role based access control system uses a centrally managed set of controls which determines how users interact with domains. The domains could be Fault management, Performance management System Admin , etc.The RBAC system controls how  users or groups of users are allowed access to the various domains and what type of access they have, i.e. View, Modify, Execute

***	END OF CHANGES	***